# ANNEX II + III:	 TECHNICAL SPECIFICATIONS + TECHNICAL OFFER

- Contract title: Supply of equipment for response to marine pollution incidents	p 1 /…

Publication reference: EuropeAid/140119/IH/SUP/ME

Columns 1-2 should be completed by the contracting authority

Columns 3-4 should be completed by the tenderer

Column 5 is reserved for the evaluation committee

Annex III - the contractor's technical offer

The tenderers are requested to complete the template on the next pages:

- Column 2 is completed by the contracting authority shows the required specifications (not to be modified by the tenderer),

- Column 3 is to be filled in by the tenderer and must detail what is offered (for example the words ‘compliant’ or ‘yes’ are not sufficient)

- Column 4 allows the tenderer to make comments on its proposed supply and to make eventual references to the documentation

The eventual documentation supplied should clearly indicate (highlight, mark) the models offered and the options included, if any, so that the evaluators can see the exact configuration. Offers that do not permit to identify precisely the models and the specifications may be rejected by the evaluation committee.

The offer must be clear enough to allow the evaluators to make an easy comparison between the requested specifications and the offered specifications.

| 1. Item number | 2. Specifications required | 3. Specifications offered | 4. Notes, remarks, ref to documentation | 5. Evaluation committee’s notes |
| --- | --- | --- | --- | --- |
| 1 | Light Oil Recovery Inflatable Booms (1000 meters): - High visibility orange colour - Material of booms: Neoprene - Freeboard no less than 300 mm. - Draft no less than 450 mm. - Section length in meters no less than 10 m and no more than 25 meters. - Weight/meter, between 3 and 4 kg/m - Reserve buoyancy (kg/m), no less than 40 kg/m. - Res. Buoyancy/weight ratio , no less than 10:1 - Waterline beam (mm) no less than 200 mm. - Ballast weight kg/m no more than 3 kg/ml - Fabric braking strain (N/5cm) no less than 2500 - Tension member, chain breaking strains (N) no less than 30 000. - Tow speed (knots) no less than 6 knots - Operating temperature from - 35 C to +70C. Training: Offer must include a four days training in Port of Bar for the operation and use of equipment and its technical characteristics. |  |  |  |
| 2 | Hydraulic - power operated Storage Reels for Booms Quantity (4) - Capacity storage of reels from 180m to 250m - Reel material Aluminium - Frame material steel painted with marine grade paints for long term corrosion protection - PVC reel cover |  |  |  |
| 3 | Sorbents (2000 kgs): - Pads, Rolls, Booms, Sock Sorbents, Oil traps, Oil Absorbents, - Non-toxic - Made of Fibrillated Polypropylene fabric - Absorb between 25x and 65x their own weight |  |  |  |
| 4 | Oil Absorbent booms (2000 meters): - Boom diameter no less than 20 cm - Boom length no less than 3 m - White colour - Weight/pack between 10 kg and 20 kg - Absorption / pack from 180 lit to 280 lit |  |  |  |
| 5 | Disc Skimmer with Hydraulic Power Pack and transfer pump Quantity (2) - Pump flow no less than 50 l/min, with automatic control - Weight, between 180 kg and 280 kg - Hydraulic pressure between 110 bar and 160 bar - Discharge pressure between 3 bar and 6 bar - Separation no less than 85% of oil and no more than 15% water - Operating draft between 35 cm and 50 cm - Single point lifting Working conditions: - Max. wave height: 2.5 meters - Air temperature from -10C to +50C - Water temperature from 0°C to +40C. Training: Offer must include a four days training in Port of Bar for the operation and use of equipment and its technical characteristics. |  |  |  |
| 6 | Air Blower Back Quantity (2) - Engine type: two stroke with gasoline fuel - Engine speed from 6000 rpm to 7000 rpm - Air flow between 800 m3/h and 1000 m3/h - Hydraulic pressure from 120 bar to 200 bar - Engine power no less than 1.5 kW - ISO standard 3744 for noise level (maximum 94dB) |  |  |  |
| 7 | Floating towable storage tank Quantity (2) - Aluminium tow bar; - Capacity minimum 50 m3 - Empty towing speed no less than 10 knots; - Towing speed when full no less than 5 knots; - Minimum two air cuffs along length of tank. Training: Offer must include a four days training in Port of Bar for the operation and use of equipment and its technical characteristics. |  |  |  |
| 8 | Portable Hydraulic Diesel Power Pack This equipment will be used as a power source for auxiliary hydraulically operated equipment such as pumps, boom reels etc. Quantity (1) - Hydraulic flow from 165 l/min to 300l/min - Working pressure from 200 bars to 300 bars. - Four point lifting rings - Automatic shutdown - Multipurpose Power Pack - Aluminium diesel fuel tank capacity no less than 25 litre Training: Offer must include a four days training in Port of Bar for the operation and use of equipment and its technical characteristics. |  |  |  |
| 9 | Oil portable storage tank Quantity (2) - Capacity, no less than 3.5 m3 and no more than 6 m3; - Collapsible tank type. Training: Offer must include a four days training in Port of Bar for the operation and use of equipment and its technical characteristics. |  |  |  |
| 10 | Portable equipment container for transporting sorbents and clean up shore equipment including sorbents kit Quantity (5) - Capacity, no less than 1.5 m3 and no more than 2 m3. - Material Plastic - Pads, Rolls, Booms, Socks Sorbents - Non-toxic - Made of Fibrillated Polypropylene Fabric - Absorb between 25x and 65x their own weight |  |  |  |
| 11 | ISO Side door container Quantity (2) - Minimum 6 m container - Double cargo doors in one end and in one side - Minimum capacity storage 2 x 250 of Ro-Boom and 2 skimmers - Floor have to be corrosion resistant - ISO certificate |  |  |  |
| 12 | Self-contained breathing apparatus Quantity (2) - Full face mask - Pressure regulator with gauge and whistle. - Pressure and flow regulator. - Max working pressure no less than 300 bar - SOLAS type approved according to Chapter II-2 of SOLAS Training: Offer must include a four days training in Port of Bar for the operation and use of equipment and its technical characteristics. |  |  |  |
| 13 | Boots Quantity (100) - Suitable to work with oil. - Rubber made or polyvinyl chloride - Waterproof. - Below knee-high |  |  |  |
| 14 | Gloves Quantity (250) - Suitable to work with oil. - Long cuffs. - Gloves for outdoor activities, with thumb and index finger touch. Gloves should have reinforcement in the palm of your hand |  |  |  |
| 15 | Protective clothing and safety helmet Quantity (200) - Disposable work suit type. - Oil resistant type. |  |  |  |
| 16 | Summer and Winter coverall suit and cap Quantity (50) - Water resistant - Two pocket on the chest - Two front cargo pocket - Colour orange and grey - Labels of the "Uprava Pomorkse Sigurnosti" |  |  |  |
| 17 | Shovels Quantity (250) - Long handles. - Made from metal (iron or aluminium). - Blades made from tempered steel or aluminium. - Square point type. |  |  |  |
| 18 | Buckets Quantity (100) - With handles. - Made from plastic. - Capacity, no less than 15 litres no more than 20 litres. |  |  |  |
| 19 | Plastic Disposable Bags Quantity (5 000) - Heavy duty type. - Capacity, no less than 25 kilograms no more than 50 kilograms. |  |  |  |
| 20 | Portable rescue & firefighting water pump Quantity (2) - Capacity no less than 2900 l/min - Hydraulic flow no less than 15 l/min - Hydraulic pressure from 150 to 250 bar - Seawater resistant - Must include all additional equipment for using (firefighting monitor, connectors between pump and fire hoses) |  |  |  |
| 21 | Multipurpose first response rigid-inflatable boat Quantity (2) Shall be able to perform functions as first response for oil spills at sea and inland waterways, search and rescue at sea and inland waterways, “port state” and “flag state” inspections at sea and inland waterways - Length overall 9 – 12m and beam minimum 3m - Dry weight from 1500 kg to 1800 kg - Full weight from 1800 kg to 2300 kg - Person capacity minimum 10 - Engine minimum 2 x 225 HP - Hull from Fiberplastic material - Tube material Neoprene- Hypalon - Tube colour: grey - Minimum speed 35 knots - Class B certificate - Compass and Steering console - Navigational console with light - Electric inflating pump - Electric panel - Bilge pump from 1500 l/min to 2500 l/min - Ladder - Fuel tank minimum 350 lit and Fuel tank integrated within the hull structure - Fuel separator - Hydraulic steering - AIS B CLASS - Fixed VHF DSC radio station - Orange rotation light and Speaker system - Labels and marking of "Uprava Pomorske Sigurnosti" - Trailer and Boat cover - SOLAS safety equipment The boat must not be a prototype. Two copies of full technical documentation are needed to be delivered in CD/DVD format. Training: Offer must include training for four skippers in area od Port of Bar. |  |  |  |
| 22 | Personal equipment for rigid-inflatable boat Quantity (8) - Open face helmet Standards EN 397, EN 966, EN 1384 EN 1385, EN 12492 and EN 1077. - SAR open face long visor - SAR protection sea wind suit with the following features: - GoreTex material and waterproof down to min10 m, with possibility neck adjustable. - Possibility MTM (Made to Measure) - Integrated Neoprane Boots. - Integrated Gloves. - Allows ventilation in non-exposed positions. - Neck seal options. - Thigh pockets - Knife pocket - Wind offshore jackets - Advanced SAR vest with the following features: - Buoyancy aid - Weight between 1 and 1.5 kgs - Colour Red - Material PU Coated Polyamide fabric. - Breaking Strength:450 kg single pull - Fastening, Interlocking Buckles, 2 or 3 Bar; - Whistle orange SOLAS approved - Artic and summer shoes - Summer clothing set shorts, polo t-shirt, light working shoes |  |  |  |
| 23 | Multipurpose-towing-first response ski yet Quantity (3) Shall be able to perform functions as first response for oil spills at sea and inland waterways, search and rescue at sea and inland waterways, “port state” and “flag state” inspections at sea and inland waterways. - Length overall between 3 and 4 m - Approved for min 2 persons - Beam minimum 1.4 m - Dry weight about 400 kg - Draft 0,3 m - Top speed min 35 knots - Engine minimum 75 kW - Fuel capacity min 60 lit. - Range minimum 90 NM - Hull Red colour - Fibreglass Cartridge from white colour - Hull shall be replaceable - Polyethylene (PE), with ≥ 3 watertight bulkheads - It shall have an aft deck with of a workplace of minimum 1.5 m2 - It has to be possible to access the waterjet from the top of the aft deck - Crew Stainless Safety Handles set - Auxiliary Electrical System AES Control Box - flashing Blue light (2 pcs) - Navigational Light: Red (1 pcs), Green (1 pcs), White (1 pcs) - Front-mounted search light on rack (2 pcs) - External 12 V Connection - QLS (Quick Load System) including Explorer Case Waterproof - Labels and markings of "Uprava Pomorske Sigurnosti" - Trailer and ventilated cover The boat must not be a prototype. Two copies of full technical documentation are needed to be delivered in CD/DVD format. Training: Offer must include training for six skippers in area od Port of Bar. |  |  |  |
| 24 | Personal equipment for the ski jet Quantity (6) - Open face helmet in line with the Standards EN 397, EN 966, EN 1384 EN 1385, EN 12492 and EN 1077. - Open face long visor - Wind offshore jacket - Advanced SAR vest - Buoyancy aid - Weight between 1 and 1.5 kgs - Colour Red - Material PU Coated Polyamide fabric. - Breaking Strength:450 kg single pull - Fastening, Interlocking Buckles, 2 or 3 Bar; - Whistle orange SOLAS approved - Gloves - Summer clothing set shorts, polo t-shirt, light working shoes |  |  |  |
| 25 | Portable VHF radio equipment Quantity (6) - SOLAS type approved intrinsically safe. - frequency range 156.025 – 157.425 MHz; - Operating temperature range -10C to +50C; - Waterproofing standards IPX8 Submersible Plus |  |  |  |
| 26 | Double cab – pick up and transport commercial vehicle The vehicle is intended to be used on all types of roads for transport of specialized equipment, including towing of boat and ski jet vessels for interventions in cases of marine pollution incidents, search and rescue operations, including access to areas affected by floods or other natural disaster, in the areas along the sea side and Lake of Skadar. Quantity (2) General requirements: - Statement that the vehicle is offered from authorized seller with an authorized service for the vehicle on the territory of Montenegro - Marks and labels clearly indicating owner (Uprava Pomorske Sigurnosti) and purpose of vehicle, special assistance number for sea 129, EU Visibility labels; - Led rotation flashing orange light, - Additional LED roof lights - Vehicle commercial warranty: min 5 years Constructional characteristics of the vehicle Engine: - Type of fuel: Diesel - Emission standard: Euro 6 - Engine volume (cm3): min 2100, max 2500 - Engine power (kW): max 130 - Average fuel consumption maximum 9 lit/100 km Transmission: - Type of transmission: Manual or Automatic - 4WD (Four-Wheel Drive) Vehicle dimension: - Length (mm): min 5000, max 5500 - Exterior width (exterior mirrors folded): max 2100 mm - Wheelbase: minimum 2700 mm - Pick-up Box Dimensions: Length at floor: min 1500, max 1620 mm Width: up to 1600 mm - Clearance: min 220 mm - Approach angle: min 28° - Wheels: min 17", Alloy - Number of seats: minimum 4 - Number of doors: minimum 4 Colour: - White with reflective self-adhesive labels Safety system: - ABS (Anti-lock Braking System) - Brake assistance - Vehicle Stability Control - Hill-start Assist Control - Downhill Assist Control - Rear differential lock - Rear-view camera Vehicle capacities: - Payload: min 1000 kg - Towing capacity: Min 2500 kg - Fuel tank capacity: min 70 lit Other equipment: - Towing hook, ready for the electrical connection to the trailer - Hard Top - Tailgate with manual or automatic locking - Manual or automatic air conditioning - Standard Airbags Packet - Tools and accessories for tyres replacement |  |  |  |