# ANNEX II + III :	 TECHNICAL SPECIFICATIONS + TECHNICAL OFFER

Contract title: Procurement of Specialised Firefighting Vehicles

Lot 3 – Grapple Truck

Ref. number: HR-BA-ME00474 – PP01

Columns 1-2 should be completed by the Project partner

Columns 3-4 should be completed by the tenderer

Column 5 is reserved for the evaluation committee

Annex III - the Contractor's technical offer

The tenderers are requested to complete the template on the next pages:

- Column 2 is completed by the Project partner shows the required specifications (not to be modified by the tenderer),

- Column 3 is to be filled in by the tenderer and must detail what is offered (for example the words “compliant” or “yes” are not sufficient)

- Column 4 allows the tenderer to make comments on its proposed supply and to make eventual references to the documentation

The eventual documentation supplied should clearly indicate (highlight, mark) the models offered and the options included, if any, so that the evaluators can see the exact configuration. Offers that do not permit to identify precisely the models and the specifications may be rejected by the evaluation committee.

The offer must be clear enough to allow the evaluators to make an easy comparison between the requested specifications and the offered specifications.

The requirements set out in the technical specifications represent the minimum technical characteristics which offered goods must satisfy, unless stated otherwise, and tenderers are not allowed to modify technical specification in any way.

For each item for which it is not explicitly stated that it is allowed to offer goods of the equal characteristics, i.e. for each item where it is not stated “or equivalent”, for the purposes of this tender documentation it is assumed that words “or equivalent” are stated, and tenderer is allowed to offer equivalent goods / goods of equivalent characteristics.

Project description

The subject of the contract is purchase of specialised firefighting vehicles for firefighting purposes. In Montenegro, the National Training Centre in Podgorica provides firefighter and rescue training across 12 municipalities: Herceg Novi, Kotor, Tivat, Budva, Bar, Ulcinj, Cetinje, Nikšić, Podgorica, Tuzi, Zeta, and Danilovgrad. The project will upgrade its infrastructure and equipment, including one medium technical firefighting vehicle, one mobile command vehicle, one grapple truck, and 13 off-road pickup vehicles (12 for municipalities and one for the National Training Centre in Podgorica). This procurement consist of four lots: Lot 01 – Medium Technical Firefighting Vehicle, Lot 02 – Command Vehicle, Lot 03 – Grapple Truck, and Lot 04 – Small Off-road Pickup Vehicles

This document refers to Lot 3 - Grapple Truck.

1. GENERAL REQUIREMENTS

1.1. The tenderer shall ensure that the functions and features of the equipment meet the listed minimum conditions and should submit equipment brochures or catalogues showing the specification and technical description of the offered equipment issued by the manufacturer.

1.2. All the equipment shall be provided complete with the necessary accessories and/or parts such as to ensure that the unit is capable of operating to the required technical and quality specifications.

1.3. The supplies shall be provided with an original set of manuals for all equipment (including technical data for offered vehicle, crane and grapple attachment). These manuals shall be supplied in English and/or Montenegrin.

1.4. The vehicle considered to include both the truck and superstructure to be supplied and must conform to the detailed Technical Specification given below.

1.5. The vehicle shall be suitable for operation in all climatic conditions in the territory of Montenegro.

1.6. The vehicle have to be fully assembled, new (produced in 2026) and unused with enclosed documentation for traffic registration and normal traffic usage. Both production and assembly must fully comply with all relevant and necessary Montenegrin laws and regulations.

1.7. All specification details listed for each item is the minimum requirements. Any improvements on the specifications or additional features offered should be clearly identified in the tenderer’s offer.

1.8. The tenderer should provide list of authorized services for the offered equipment, in the territory of Montenegro, with addresses and contact telephone, issued by the manufacturer or the authorized representative of the manufacturer for the territory of Montenegro.

1.9. The Tenderer shall provide statement or certificate issued by the manufacturer/body builder confirming that the offered configuration is certified, with the mandatory indication of the model of the equipment being offered.

1.10 Homologation certificate or proof that the vehicle can be registered in Montenegro.

1.11 For each offered item marked as “or equivalent”, the tenderer shall provide documentary evidence demonstrating that the proposed solution meets the same or superior functional and performance requirements.

2. SERVICE PROVISIONS

2.1. Delivering and installation: The Contractor will be responsible for installation, configuration and start-up of the supplies delivered in accordance with the directions provided by the Contracting Authority. Delivery address for the equipment is the National Training Centre, Bulevar Josipa Broza Tita, Podgorica, Montenegro. The contractor shall agree the delivery and installation schedule with the Contracting Authority.

2.2. Testing: Items shall be tested during the takeover event, where all the required specifications and features shall be accounted for, and full functionality of the items shall be presented.

3. WARRANTY CONDITIONS

3.1. Warranty is as defined in Article 32 of the Special Conditions. Warranty for the vehicle and superstructure is 12 months, starting from the date of issuance of Provisional Acceptance Certificate

3.2. Commercial warranty only for superstructure is minimum 24 months, starting from the date of issuance of Final Acceptance Certificate. This additional warranty period is separately defined directly in description of the item.

3.3. The Contractor is responsible for the provision of “maintenance” and “repair” of the supplies within the warranty period without any charge.

3.4. The contractor shall guarantee the proper operation of the supplies. In the event of failure of the supplies' operability during the warranty period, the contractor shall restore it, and in case of impossibility to do so, shall replace it with equivalent supplies in terms of the technical characteristics. The contractor at his own expense shall perform the replacement.

4. VISIBILITY REQUIREMENTS

The Contractor shall take the necessary measures to ensure the visibility. These measures must comply with the Programme visibility rules available at: https://interreg-hr-ba-me.eu/documents/implementation/ . All equipment shall have a fixed durable label, approved by the contracting authority.

LOT 3: GRAPPLE TRUCK

| 1. Item Number | 2. Specifications Required | 3. Specifications Offered | 4. Notes, remarks, ref to documentation | 5. Evaluation Committee’s decision (Y/N) |
| --- | --- | --- | --- | --- |
| 3.1 | GRAPPLE TRUCK Truck with superstructure three-sided tipper box with a crane and a grab for lifting the cargo Quantity: 1 truck |  |  |  |
|  | - Year of manufacture 2026. year - gross vehicle weight maximum 14t - payload capacity of the vehicle without superstructure, minimum 9,500 kg - engine: minimum Euro 6, or equivalent - fuel: diesel - engine volume: minimum 5000 ccm - engine power: minimum 175 kw - engine brake included - gearbox: manual with a minimum of 6 gears + reverse gear, - power take off from the gearbox - drive configuration: 4x2 on the rear wheels - double wheels on the rear axle - wheelbase maximum 3800mm - front leaf spring suspension, double-acting hydraulic shock absorbers, stabilizer. |  |  |  |
|  | -The rear suspension must be designed to ensure vehicle stability at full load, safe movement and operation of the vehicle with a built-in body, crane and tipper body, in accordance with applicable regulations and manufacturer s approval. Air, mechanical or combined suspension is permitted. The proposed solution must be approved by the manufacturer for the vehicle and upgrading parts. |  |  |  |
|  | - driver's seat with air suspension - air conditioner - electric windows - central locking with remote control - fog lights - spare wheel with carrier - number of seats: 3 - cabin color: white - body color: White with red and yellow fluorescent stripes along the sides and on the hood |  |  |  |
|  | UPGRADE: Three-sided tipper box with crane and grab for lifting cargo Three-sided tipper box : - internal length of tipper box: minimum 3,400 mm - inner width of tipper box: maximum 2,450 mm - height of the front side panel: minimum 500 mm - front side panel thickness: minimum 3 mm - height of side panels and rear sides panel: minimum 500 mm - thickness of side panels and rear sides panel : minimum 3 mm - tipper box volume: minimum 4m3 - side panels: one-piece - opening of the side panels: by folding - opening of the back panel: tilt opening (automatic) - thickness of the floor panel of the tipper box: minimum 4mm - cargo box lifting system: centrally located telescopic hydraulic cylinder - drive of the hydraulic pump via the power take-off from the vehicle's gearbox - load tipping control: valve (Joystick) in the vehicle cabin located laterally in relation to the driver's seat Load lifting crane: - Hydraulic outlets on top of crane for the grab rotator - Rotator with 6-pole grab - Lifting torque of the crane minimum: 10 tm - Maximum horizontal hydraulic reach 9m - The load capacity of the crane at the maximum hydraulic reach 850 kg - Turning angle: minimum 400 degrees - Control from the ground and with remote wireless controls - Place of installation of the crane: behind the cab — Stabilizers opening range minimum 4500 mm - Manual, hydraulic or electro-hydraulic control of stabilizers, provided that the system ensures stable and safe operation of the crane, overload protection and compliance with applicable regulations, standards and homologation requirements for the vehicle and crane. - Hydraulic device for overload - Counter of working hours |  |  |  |