# CORRIGENDUM NO. 1.

For Procurement of Specialized firefighting vehicles for Protection and rescue services

PUBLICATION REF.: 02/2025

Based on clarification requests by economic operators the following changes are made by this Corrigendum:

Within category Safety and Assistance Systems request ”Automatic emergency braking” was deleted

The interested economic operator submitted the request and stated that Automatic emergency breaking functions within ADAS system, which consists of a lane keeping system, lane departure system and other systems, and that it uses autonomous breaking and stopping when system recognized danger, and that during emergency situations vehicles are often required to overtake full lines, approach front vehicle more than expected etc, and that such system should not be requested for the emergency  vehicles. The explanation and the request were found to be justified, so this request is being deleted.

Within category Wheels, Tires, and Exterior: request for wheels was changed to: aluminum or steel rims, minimum 18″

The interested economic operator submitted the request and stated amendment would improve the vehicles suitability for off road use and reflect common industry practice. The explanation and the request were found to be justified, so this request is being amended.

The new Technical specifications, with integrated changes are provided below and replace the previous version published as part of the tender dossier.

# ANNEX II + III:	 TECHNICAL SPECIFICATIONS + TECHNICAL OFFER

Contract title: Procurement of Specialized firefighting vehicles for Protection and rescue services

p 1 /…

Publication reference: 02/2025

Columns 1-2 should be completed by the contracting authority

Columns 3-4 should be completed by the tenderer

Column 5 is reserved for the evaluation committee

Annex III - the contractor's technical offer

The tenderers are requested to complete the template on the next pages:

- Column 2 is completed by the contracting authority shows the required specifications (not to be modified by the tenderer),

- Column 3 is to be filled in by the tenderer and must detail what is offered (for example the words ‘compliant’ or ‘yes’ are not sufficient)

- Column 4 allows the tenderer to make comments on its proposed supply and to make eventual references to the documentation

The eventual documentation supplied should clearly indicate (highlight, mark) the models offered and the options included, if any, so that the evaluators can see the exact configuration. Offers that do not permit to identify precisely the models and the specifications may be rejected by the evaluation committee.

The offer must be clear enough to allow the evaluators to make an easy comparison between the requested specifications and the offered specifications.

| 1. Item number | 2. Specifications required | 3. Specifications offered | 4. Notes, remarks, ref to documentation | 5. Evaluation committee’s notes |
| --- | --- | --- | --- | --- |
| 1 | FIREFIGHTING VEHICLE 4×4 PICKUP ”CREW CAB” – 6 PIECES Vehicles with following minimal characteristics: Emission standard: minimum EURO 6d Condition: new, unused vehicle Engine and Drivetrain: Engine displacement: max. 2,000 cm³ Power: ≥ 120 kW (164 hp) at 3,600 rpm Fuel: diesel Transmission: 6-speed manual (+ 1 reverse) Drivetrain: permanent or selectable 4×4 with transfer case Differential lock: mechanical Towing capacity: ≥ 3,500 kg Payload capacity for base vehicle: ≥ 1,000 kg Wheels, Tires, and Exterior: Wheels: aluminum or steel rims, minimum 18″ Spare wheel: full-size, matching the main wheels Winch: mounted in the front bumper Fog lights: front Body color: white, with reflective red film covering ≥ 60 % of each side Cargo Area: Cargo area cover: watertight Platform load capacity: ≥ 200 kg (excluding water/foam) Firefighting Module: Adapted to the dimensions of the vehicle’s load compartment Water tank: stainless steel, capacity ≥ 200 L, with DN52 Storz coupling for filling, plus a level indicator and vent Foam tank: stainless steel or HD plastic, capacity ≥ 20 L High-pressure pump: flow ≥ 42 L/min at ≥ 150 bar; driven by an independent diesel engine ≥ 14 kW, with electric start and manual backup Foam proportioning and mixing controls located on the control panel Hose reel: manual winding, with at least 50 m of hose rated for ≥ 180 bar Nozzle: adjustable for full jet, spray jet, and foam Stretcher for injured persons: fold-away, integrated Telescopic ladder: length ≥ 3.5 m Signaling and Communication: Roof-mounted rotating beacon with integrated electronic sirens Additional lights: front and rear LED Safety and Assistance Systems: ABS with stability control (ABS, ESC, and TCS) Hill start assist / hill descent control (HSA / HDC) Airbags Comfort and Ergonomics: Power windows on all doors Mirrors: electrically adjustable, heated, folding Air conditioning Audio system: radio, USB, Bluetooth, or equivalent |  |  |  |