Predmer i predra	un                   Sewage sewer system in Berane

=======================================================================

REKAPITULACIJA

----------------------------------------------------------------------------------------------------------------------

Vrsta posla                                                                             Ukupno                  Strana

----------------------------------------------------------------------------------------------------------------------

Preliminary works                                                               ____________                       1

Earth works                                                                       ____________                       2

Concrete works                                                                  ____________                       9

Reinforcing works                                                              ____________                      12

Installation works                                                              ____________                      13

Other works                                                                      ____________                      16

----------------------------------------------------------------------------------------------------------------------

UKUPNO:                                                                           ____________

____________________________________________________________________________________________

REKAPITULACIJA

Predmer i predračun                   Sewage sewer system in Berane                                        Strana   1

=======================================================================

Poz.    Opis pozicije                                                     J.M.     Količina    Jed. cena        Ukupno

=======================================================================

01      Preliminary works

----------------------------------------------------------------------------------------------------------------------

01.001  Marking of the route, trench and pipeline alignment

control during construction and all necessary geodetic

survey for as-built project.

Route marking and alignment control to be realized

in accordance with goemteric elements of the route,

given by main design.

For as-built project, geodetic survey to be performed

using instruments with adequate accuracy for this kind

of work.

Unit price covers all necessary work and equipment in

accordance with technical regulations.

Calculation per m' of pipeline.

m'     20678.00 x____________ =____________

----------------------------------------------------------------------------------------------------------------------

Ukupno:                                                                                                          ____________

____________________________________________________________________________________________

Preliminary works

Predmer i predračun                   Sewage sewer system in Berane                                        Strana   2

=======================================================================

Poz.    Opis pozicije                                                     J.M.     Količina    Jed. cena        Ukupno

=======================================================================

02      Earth works

----------------------------------------------------------------------------------------------------------------------

02.001  Mechanical and manual excavation of the trench

in all categories of soil (estimated category III and

IV).

Contractor must check terrain characteristics before

preparing tender.

Unit price covers all necessary work and material,

including necessary attention to preserve current

installations along designed pipeline and possible trench

supporting and pumping water out of the trench.

There is also possibility of wastewater presence

while excavation.

Calculation per m3 of properly and well excavated

soil with loading and transportation to landfill, up to

10 km distance.

Excavations will be carried out in asphalted roads

and there is possibility of wastewater presence on

some sections.

Looseness ratio to be covered with unit price.

Excavation up to 2m depth in accordance with

evidence of quantities.

Collectors

38639.63-2926.42=                                                  35713.21

House connections

5668.75=                                                                   5668.75

==========

m3     41381.96 x____________ =____________

____________________________________________________________________________________________

Earth works

Predmer i predračun                   Sewage sewer system in Berane                                        Strana   3

=======================================================================

Poz.    Opis pozicije                                                     J.M.     Količina    Jed. cena        Ukupno

=======================================================================

02.002  Mechanical and manual excavation of the trench

in all categories of soil (estimated category III and

IV).Contractor must check terrain characteristics before

preparing tender.

Unit price covers all necessary work and material,

including necessary attention to preserve current

installations along designed pipeline and possible trench

supporting and pumping water out of the trench.

There is also possibility of wastewater presence

while excavation.Calculation per m3 of properly and well excavated

soil with loading and transportation to landfill, up to

10 km distance.

Excavations will be carried out in asphalted roads

and there is possibility of wastewater presence on

some sections.

Looseness ratio to be covered with unit price.

Excavation from 2 to 4m of depth in accordance

with evidence of quantities.

Collectors

12119.39-803.07=                                                  11316.32

House connections

931.20=                                                                     931.20

==========

m3     12247.52 x____________ =____________

----------------------------------------------------------------------------------------------------------------------

02.003  Mechanical and manual excavation of the trench

in all categories of soil (estimated category III and

IV). Contractor must check terrain characteristics before

preparing tender. Unit price covers all necessary work and material,

including necessary attention to preserve current

installation along designed pipeline and possible trench

supporting and pumping water out of the trench.

There is also possibility of wastewater presence

while excavation.

Calculation per m3 of properly and well excavated

soil with loading and transportation to landfill, up to

10 km distance.

Excavations will be carried out in asphalted roads

and there is possibility of wastewater presence on

some sections.

Looseness ratio to be covered with unit price.

Excavation from 4 to 6m of depth in accordance

with evidence of quantities.

Collectors

417.23=                                                                  417.23

==========

m3       417.23 x____________ =____________

____________________________________________________________________________________________

Earth works

Predmer i predračun                   Sewage sewer system in Berane                                        Strana   4

=======================================================================

Poz.    Opis pozicije                                                     J.M.     Količina    Jed. cena        Ukupno

=======================================================================

02.004  Mechanical and manual excavation of the trench

in all categories of soil (estimated category III and

IV).

Supervisor will define on site category of excavated

soil.

Contractor must check terrain characteristics before

preparing tender.

Unit price covers all necessary work and material,

including necessary attention to preserve current

installation along designed pipeline and possible trench

supporting and pumping water out of the trench.

Calculation per m3 of properly and well excavated soil

with dumping excavated material in sufficient distance

from trench, to prevent disruption of the communication

along trench while placeing pipes or backfilling trench

Excavations will be carried out in roads without asphalt.

Excavation up to 2m depth in accordance with evidence

of quantites.

Collectors

Pipeline D0                                                                1609.04

1609.04=

Pipeline L1                                                                 396.48

396.48=

Pipeline L1.1                                                               335.58

335.58=

Pipeline L1.3.3                                                             585.32

585.32=                                                                 ==========

m3      2926.42 x____________ =____________

____________________________________________________________________________________________

Earth works

Predmer i predračun                   Sewage sewer system in Berane                                        Strana   5

=======================================================================

Poz.    Opis pozicije                                                     J.M.     Količina    Jed. cena        Ukupno

=======================================================================

02.005  Mechanical and manual excavation of the trench

in all categories of soil (estimated category III and

IV). Supervisor will define on site category of excavated

soil. Contractor must check terrain characteristics before

preparing tender.

Unit price covers all necessary work and material,

including necessary attention to preserve current

installation along designed pipeline and possible trench

supporting and pumping water out of the trench.

Calculation per m3 of properly and well excavated soil

with dumping excavated material in sufficient distance

from trench, to prevent disruption of the communication

along trench while placeing pipes or backfilling trench

Excavations will be carried out in roads without asphalt.

Excavation from 2 to 4m of depth in accordance with

evidence of quantites.

Collectors

Pipelinie D0

536.73=                                                                     536.73

Pipeline L1

110.67=                                                                     110.67

Pipeline L1.1

24.65=                                                                       24.65

Pipeline L1.3.3

131.02=                                                                     131.02

==========

m3       803.07 x____________ =____________

----------------------------------------------------------------------------------------------------------------------

02.006  Additional excavation for manholes aftter trench

excavation. Excavation will be carried out in same material

as trench excavation.

Unit price covers all necessary work and material,

including necessary attention to preserve current

installations along designed pipeline and possible trench

supporting and pumping water out of the trench.

There is also possibility of wastewater presence

while excavation.

Calculation per m3 of properly and well excavated

soil with loading and transportation to landfill, up to

10 km distance.

Looseness ratio to be covered with unit price.

Calculation per m3 in accordance with evidence of

quantites.

Collectors

7791.13=                                                                   7791.13

==========

m3      7791.13 x____________ =____________

____________________________________________________________________________________________

Earth works

Predmer i predračun                   Sewage sewer system in Berane                                        Strana   6

=======================================================================

Poz.    Opis pozicije                                                     J.M.     Količina    Jed. cena        Ukupno

=======================================================================

02.007  Manual excavation in all categories of soil, near

installations that crosses with designed pipeline, before

and while trench excavation.

Calculation per m3 of properly and well excavated

trench, including material dumping in sufficient distance

from trench, to prevent disruption of the communication

along trench.

m3       100.00 x____________ =____________

----------------------------------------------------------------------------------------------------------------------

02.008  Manual finishing and planninf of the trench bottom after

mechanical excavation.

Trench finishing to be carried out with carefully selected

material from excavation, with accuracy +/-3 cm regarding

designed pipeline.

On finished trench bottom to be placed pipe bed.

Unit price covers all necessary work.

Calculation per m2 of bottom trench.

Collectors

17683*0.80=                                                         14146.40

House connections

2995*0.80=                                                             2396.00

==========

m2     16542.40 x____________ =____________

----------------------------------------------------------------------------------------------------------------------

02.009  Making of pipe bed for sewage pipes.

Pipe bed to be made of sand, with 10 cm thickness

below, above and around pipe in total trench width.

Pipe bed can also be made of sieved material from

excavation, excapt that fractions larger than 8-10mm

shouldn't come in touch with pipeline.

Calculation per m3 of quality carried out pipe bed

in accordance with evidence of quantities.

Collectors

6363.62=                                                                   6363.62

House connections

879.39=                                                                     879.39

==========

m3      7243.01 x____________ =____________

____________________________________________________________________________________________

Earth works

Predmer i predračun                   Sewage sewer system in Berane                                        Strana   7

=======================================================================

Poz.    Opis pozicije                                                     J.M.     Količina    Jed. cena        Ukupno

=======================================================================

02.010  Procurement, transport and placing of sub-base

layer, after installation of pipes and pipe bed.

Sub-base layer to be placed instead of trench

material.

Sub-base layer to be placed in layers not thicker

than 30cm with compaction to the required module of

compressibility, same os for base layer for atphalt

or concrete.

First layer backfilling to be carried out manually.

Further backfilling can be carried out mechanically

with the condition that while backfilling, machines don't

cross trench with installed and tested pipeline and

layers shouldn't be thicker than 30 cm with required

compaction.

Calculation per m3 of placed and compacted sub-base

layer.

Collectors

40319.23=                                                                 40319.23

House connections

5606.52=                                                                   5606.52

==========

m3     45925.75 x____________ =____________

----------------------------------------------------------------------------------------------------------------------

02.011  Trench backfilling with material from excavation, after

pipeline installation and paceing of pipe bed.

First layer of backfilling to be carried out manually.

Further backfilling to be carried out mechanically,

with the condition that while trench backfilling machines

don't cross trench with installed and tested pipeline and

layers shouldn't be thicker than 50 cm with required

compaction.

Surplus material to outspread in surrounding terrain.

Calculation per m3 of placed and compacted material

from excavatin in accordance with evidence of quantities.

Collectors

3032.72=                                                                   3032.72

==========

m3      3032.72 x____________ =____________

----------------------------------------------------------------------------------------------------------------------

02.012  Truck load, transport, unload and outspread of

surplus material in landfill from pipeline L1.3.3.

Calculation per m3.

Pipeline L1.3.3

140.67=                                                                     140.67

==========

m3       140.67 x____________ =____________

____________________________________________________________________________________________

Earth works

Predmer i predračun                   Sewage sewer system in Berane                                        Strana   8

=======================================================================

Poz.    Opis pozicije                                                     J.M.     Količina    Jed. cena        Ukupno

=======================================================================

02.013  Increas of current and construction of embankment

along right bank of river Lim, on section of pipeline D0.

Material for embankment is with gravelly composition,

that was created with the deposition of river sediment

along riverbed.

Unit price covers excavation, truck load and transport

up to 5km.

Embankment to be carried out in accordance with

project details.

Calculation per m3 of excavated, transported and placed

material.

m3      9289.97 x____________ =____________

----------------------------------------------------------------------------------------------------------------------

02.014  Mechanic excavation of embankment fundament

trench. Embankment to be carried out with crushed

stones.

Excavation to be carried out up to 2 m length, while

excavated material to be used for embankment.

Calculation per m3 of excavated and placed material.

170*1.00*1.80=                                                        306.00

==========

m3       306.00 x____________ =____________

----------------------------------------------------------------------------------------------------------------------

02.015  Procurement of the stone with tranport and coating of

embankment slope with one layer stacked stone, with

minimum dimensions 70x50x40cm, aiming to build

embankment in endangered sections.

Stacking stones to be carried out manually and with

machines (adequante machines) along formed embankment

in accordance with designed alignment and project details.

Stacking of the stones to be carried out carefully, to

ensure stability of riverbank ond protection from high water.

Calculatin per m3 of placed stone.

170*7.00=                                                              1190.00

==========

m3      1190.00 x____________ =____________

----------------------------------------------------------------------------------------------------------------------

Ukupno:                                                                                           ____________

____________________________________________________________________________________________

Earth works

Predmer i predračun                   Sewage sewer system in Berane                                        Strana   9

=======================================================================

Poz.    Opis pozicije                                                     J.M.     Količina    Jed. cena        Ukupno

=======================================================================

03      Concrete works

----------------------------------------------------------------------------------------------------------------------

03.001  Constructing manholes from concrete rings with lower

slab and manhole bottom, with concrete MB30.

Simultaneously with manhole bottom, clamp made of

polyester and relevant diameter with NRB quality integrated

rubber seals should be installed, in order to prevent

inflitration of external water after pipeline installation.

Unit price covers all necessary work and metarila, including

necessary formwork.

Works to be carried out in accordance with regulations

for these kind of works and details from project.

Calculation per m3 of casted concrete.

297.45+549.04=                                                     846.49

==========

m3       846.49 x____________ =____________

----------------------------------------------------------------------------------------------------------------------

03.002  Procurement, transport and casting MB30 concrete into

reinforcing concrete upper slab and wreath below manhole

slab. Manholes made of concrete rings.

Concrete slab is 20cm thick and to be carried out in

accordance with project details.

Unit price covers all necessary work and material, including

necessary formwork.

Calculation per m3 of casted concrete in accordance with

evidence of quantities.

260.09+197.26=                                                      457.35

==========

m3       457.35 x____________ =____________

----------------------------------------------------------------------------------------------------------------------

03.003  Procurement, transport and installation of reinforced

concrete rings with diameter 1000mm and length L=1000mm

with integrated NRB quality rubber seal.

Rings should be placed on manhole base, to form manhole

body.

Unit price covers all necessary work and material for

concrete ring installation, including necessary chiseling

for iron steps, as well as mending after iron steps

installation, pipe shortening and other work needed to

form manhole walls, to prevent infiltration of external

water.

Calculation per piece of installed RC rings.

piec      1503.00 x____________ =____________

____________________________________________________________________________________________

Concrete works

Predmer i predračun                   Sewage sewer system in Berane                                        Strana  10

=======================================================================

Poz.    Opis pozicije                                                     J.M.     Količina    Jed. cena        Ukupno

=======================================================================

03.004  Construction of stability bench from MB30 reinforced

concrete in Lim riverbed where pipeline D0 crosses

river. Bnech has length L=60m, hight and width 70x70cm.

For one meter of bench is necessary 0.49m3 of concrete

and 18 kg of concrete iron Q424.

Unit price covers all necessary work and material, including

necessary formwork and earth work for excavation and

cofferdam.

Works to be carried out in accordance with regulations for

these kind of works and project details.

Calculation per m3 of casted concrete.

60*0.70*0.70=                                                          29.40

==========

m3        29.40 x____________ =____________

----------------------------------------------------------------------------------------------------------------------

03.005  Construction of a section of the stability bench made

of concrte in river Lim where pipeline D0 crosses river.

Bench has length L=60m, hight 50cm and width 15m.

Bench should be made of MB25 concrte and crushed

stone in accordance with project details.

From total volume, 60% is crushed stone and 40% concrete.

Stone must be cleaned before placement. Also stones

larger than 100mm from river sediment can be used.

Unit price covers all necessary work and material, including

necessary formwork and earth works for excavation and

cofferdam. Workd to be carried out in accordance with regulations for

these kind of works and project details.

Calculation per m3 of casted concrete.

60*0.50*15=                                                            450.00

==========

m3       450.00 x____________ =____________

----------------------------------------------------------------------------------------------------------------------

03.006  Construction of concrete trench made of MB30

reinforced concrete, where pipeline GRP-B2 Dn400mm

should be placed.Concrete trench to be constructed in croos point where

pipeline D0 goes below riverbed.

Concrete trench has length L=90m, between manhole

D0-RO3 i D0-RO4.

For one meter of concrete trench it is necessary 0.89m3

of concrete and 54 kg of concrete iron Q424.

Unit price covers all necessary work and material, including

necessary formwork and earth works for excavation and

cofferdam in riverbed.

Works to be carried out in accordance with regulations for

these kind of works and project details.

Calculation per m3 of casted concrete.

90*0.89=                                                                     80.10

==========

m3        80.10 x____________ =____________

____________________________________________________________________________________________

Concrete works

Predmer i predračun                   Sewage sewer system in Berane                                        Strana  11

=======================================================================

Poz.    Opis pozicije                                                     J.M.     Količina    Jed. cena        Ukupno

=======================================================================

03.007  Construction of concrete trench made of MB30

reinforced concrete, where pipeline GRP-B2 DN250mm

should be placed.

Concrete trench to be constructed in croos point where

pipeline L1.1 goes below Makva streambed.

Concrete trench has length L=90m.

For one meter of concrete trench it is necessary 0.50m3

of concrete and 43 kg of concrete iron Q424.

Unit price covers all necessary work and material, including

necessary formwork and earth works for excavation.

Works to be carried out in accordance with regulations for

these kind of works and project details.

Calculation per m3 of casted concrete.

7*0.50=                                                                    3.50

==========

m3         3.50 x____________ =____________

----------------------------------------------------------------------------------------------------------------------

Ukupno:                                                                                           ____________

____________________________________________________________________________________________

Concrete works

Predmer i predračun                   Sewage sewer system in Berane                                        Strana  12

=======================================================================

Poz.    Opis pozicije                                                     J.M.     Količina    Jed. cena        Ukupno

=======================================================================

04      Reinforcing works

----------------------------------------------------------------------------------------------------------------------

04.001  Procurement, transport, cutting, bending and

placement of concrete iron.

Unit price covers all necessary work and material

for proper placement of concrete iron in accordance

with project details.

88905+5940=                                                       94845.00

==========

kg     94845.00 x____________ =____________

----------------------------------------------------------------------------------------------------------------------

Ukupno:                                                                                           ____________

____________________________________________________________________________________________

Reinforcing works

Predmer i predračun                   Sewage sewer system in Berane                                        Strana  13

=======================================================================

Poz.    Opis pozicije                                                     J.M.     Količina    Jed. cena        Ukupno

=======================================================================

05      Installation works

----------------------------------------------------------------------------------------------------------------------

05.001  Procurement, transport and installation of sewage pipes

made of centrifugal polyester, in accordance with standard

EN1796 and EN14364, load capacity Sn5000, with clamps

and NBR quality rubber seals.

Pipes to be installed in previous placed pipe bed made of

sand in accordance with project details.

Unit price covers all necessary work and material for fully

and proper pipe installation, in accordance with regulations

for these kind of works.

Calculation per m' of installed and from Supervisor accepted

pipes made of centrifugated polyester.

DN500 external diameter 509.60mm, inner diameter 530mm

m'      1858.00 x____________ =____________

----------------------------------------------------------------------------------------------------------------------

05.002  Procurement, transport and installation of sewage pipes

made of centrifugal polyester, in accordance with standard

EN1796 and EN14364, load capacity Sn5000, with clamps

and NBR quality rubber seals.

Pipes to be installed in previous placed pipe bed made of

sand in accordance with project details.

Unit price covers all necessary work and material for fully

and proper pipe installation, in accordance with regulations

for these kind of works.

Calculation per m' of installed and from Supervisor accepted

pipes made of centrifugated polyester.

evi se ugra|uju na predhodno izra|enoj

DN400 external diameter 427mm, inner diameter 410.00mm

m'       213.00 x____________ =____________

----------------------------------------------------------------------------------------------------------------------

05.003  Procurement, transport and installation of sewage pipes

made of centrifugal polyester, in accordance with standard

EN1796 and EN14364, load capacity Sn5000, with clamps

and NBR quality rubber seals.

Pipes to be installed in previous placed pipe bed made of

sand in accordance with project details.

Unit price covers all necessary work and material for fully

and proper pipe installation, in accordance with regulations

for these kind of works.

Calculation per m' of installed and from Supervisor accepted

pipes made of centrifugated polyester.

DN300 external diameter 324mm, inner diameter 310.40mm

m'      4606.00 x____________ =____________

____________________________________________________________________________________________

Installation works

Predmer i predračun                   Sewage sewer system in Berane                                        Strana  14

=======================================================================

Poz.    Opis pozicije                                                     J.M.     Količina    Jed. cena        Ukupno

=======================================================================

05.004  Procurement, transport and installation of sewage pipes

made of centrifugal polyester, in accordance with standard

EN1796 and EN14364, load capacity Sn5000, with clamps

and NBR quality rubber seals. Pipes to be installed in previous placed pipe bed made of

sand in accordance with project details.

Unit price covers all necessary work and material for fully

and proper pipe installation, in accordance with regulations

for these kind of works. Calculation per m' of installed and from Supervisor accepted

pipes made of centrifugated polyester.

DN250 external diameter 272mm, inner diameter 260.20mm

m'     11005.00 x____________ =____________

----------------------------------------------------------------------------------------------------------------------

05.005  Procurement, transport and installation of sewage pipes

made of centrifugal polyester, in accordance with standard

EN1796 and EN14364, load capacity Sn5000.

Pipe length is L=6.00m, pipes are shiped with clamp

for concreting on one end, while on other end with

clamp for transition from GRP pipes into PVC and

also are shiped with a cover from PDV DN 200mm.

clamps are with NBR quality rubber seals.

Pipes to be installed in previous placed pipe bed made of

sand in accordance with project details.

Unit price covers all necessary work and material for fully

and proper pipe installation, in accordance with regulations

for these kind of works.  Calculation per piece of installed and from Supervisor

accepted pipes made of centrifugated polyester.

DN200 external diameter 220.00mm, inner diameter 210.00mm

piec       500.00 x____________ =____________

----------------------------------------------------------------------------------------------------------------------

05.006  Procurement, transport and placement of covers with

ductile iron frame (according to regulation EN124).

Covers are coated with non-toxic hidrosobil black paint,

non polluting by BS3416. Covers are circular, 60cm diameter of aperture, with

load capacity of 400 kN (class D400) and joint connection

between frame and cover with rubber seal or conical fit.

Unit price covers all necessary work and material for

quality installation of covers, in accordance with project

details; drilling holes and anchoring of cover frame into

RC slab after concreting.

Calculation per pice of installed cover.

piec       661.00 x____________ =____________

----------------------------------------------------------------------------------------------------------------------

05.007  Procurement, transport and placement of iron steps

into manholes in accordance with JUS M.J6.285.

Iron steps to be installed in accordanc with project

details.Calculation per piece of installed iron step.

1317+389=                                                                  1706.00

==========

piec      1706.00 x____________ =____________

____________________________________________________________________________________________

Installation works

Predmer i predračun                   Sewage sewer system in Berane                                        Strana  15

=======================================================================

Poz.    Opis pozicije                                                     J.M.     Količina    Jed. cena        Ukupno

=======================================================================

05.008  Reconstruction of existing pipeline for sanitary water

along street where is designed pipeline D1.2

(pipeline is from asbestos cement pipes with diameter

100mm, tyoe C).

Existing pipes should be replaced with new pipes PEHD

DN110mm PE100 NP10.

Unit price covers procurement and installation of necessary

fittings and connecting elements, including reconnection

of all house connections, as well as necessary earth, concrete,

reinforcing and other works for replacment of pipeline.

Because pipelie route is not fully known, all works including

necessary technical solutions and details, to be in accordance

with Berane Water Utility representatives and Supervisor.

Calculation per meter of replaced pipeline.

m'       350.00 x____________ =____________

----------------------------------------------------------------------------------------------------------------------

Ukupno:                                                                                                        ____________

____________________________________________________________________________________________

Installation works

Predmer i predračun                   Sewage sewer system in Berane                                        Strana  16

=======================================================================

Poz.    Opis pozicije                                                     J.M.     Količina    Jed. cena        Ukupno

=======================================================================

06      Other works

----------------------------------------------------------------------------------------------------------------------

06.001  Pipeline testing with free flow under test pressure in

accoradnce with regulations for sewer pipe testing EN1610,

after installation and backfilling with first sub-base layer.

Calculation per m' of tested pipeline.

m'     20678.00 x____________ =____________

----------------------------------------------------------------------------------------------------------------------

06.002  Reconnection of disconnected secondary pipelines for water

supply, which are not recorded in cadastre of installations.

Pipe diameters are up to 3'' or made of PE up to

diameter DN90. Unit price covers additional manual excavation, manual

backfilling after reconnecting, as well as necessary work

and material for reconnection. Reconnection to be carried out with the presence of

Berane Water Utility representative.

Calculation per piece of reconnections.

piec        50.00 x____________ =____________

----------------------------------------------------------------------------------------------------------------------

06.003  Reconnection of disconnected electrical cables which are

not recorded in cadastre of installations.

Unit price covers additional manual excavation, manual

backfilling after reconnecting, as well as necessary work

and material for reconnection.

Calculation per piece of reconnections.

piec         5.00 x____________ =____________

----------------------------------------------------------------------------------------------------------------------

06.004  Reconnection of disconnected TT cables which are

not recorded in cadastre of installations.

Unit price covers additional manual excavation, manual

backfilling after reconnecting, as well as necessary work

and material for reconnection.

Calculation per piece of reconnections.

piec        10.00 x____________ =____________

----------------------------------------------------------------------------------------------------------------------

06.005  Cutting and breaking asphalt with pickamer with truck

load and transportation to landfill, up to 10 km distance.

Asphalt edge to cut straight, 40 cm wider from left and

right side of the trench, to prevent damaging of the

remain asphalt while excavation of the trench.

It is estimated that for 1 m of pipeline will be damaged

2m2 of asphalt. Assumed asphalt thickness is different, from 6cm to

16cm (6+6+4). Based on detailed site inspection, it is estimated number

of layers, as well as thickness of existing asphalt

regarding new designed collectors, which is given in

evidence of quantities. Unit price covers procurement and placement of new

asphalt on sub-base layer which is compacted to required

module of compressibility, as well as scraping of existing

asphalt if its necessary, after trench backfilling.

Calculation per tons of installed asphalt BNs22.

tons      8732.00 x____________ =____________       ____________________________________________________________________________________________

Other works

Predmer i predračun                   Sewage sewer system in Berane                                        Strana  17

=======================================================================

Poz.    Opis pozicije                                                     J.M.     Količina    Jed. cena        Ukupno

=======================================================================

06.006  Cutting and breaking asphalt with pickamer with truck

load and transportation to landfill, up to 10 km distance.

Asphalt edge to cut straight, 40 cm wider from left and

right side of the trench, to prevent damaging of the

remain asphalt while excavation of the trench.

It is estimated that for 1 m of pipeline will be damaged

2m2 of asphalt. Assumed asphalt thickness is different, from 6cm to

16cm (6+6+4). Based on detailed site inspection, it is estimated number

of layers, as well as thickness of existing asphalt

regarding new designed collectors, which is given in

evidence of quantities.  Unit price covers procurement and placement of new

asphalt on sub-base layer which is compacted to required

module of compressibility, as well as scraping of existing

asphalt if its necessary,after trench backfilling.

Calculation per tons of installed asphalt ABs11.

tons      1673.00 x____________ =____________

----------------------------------------------------------------------------------------------------------------------

06.007  Geodetic survey and drafting of the cadastre for installed

sewer network and objects. Works to be performed in accordance with Law and

Regulations that define drafting of underground installations

cadastre.  Data should be processed in DWG format and GIS

platform, used by Water Utility Berane.

m'     20677.00 x____________ =____________

----------------------------------------------------------------------------------------------------------------------

06.008  Cutting and breaking with pickamer concrete (pavements)

along new designed pipeline with truck load and transport

to landfill, up to 10km distance. Concrete edge to cutt in way to prevent damaging of

remained surface, while excavation of the trench.

It is estimated that concrete thickness is 20cm.

Cutting width is 1.60cm. Unit price covers procurement and casting of MB30

concrete on sub-base layer, which is compacted to

required module of compressibility, after trench backfilling.

New casted concrete layer has same thickness as

existing layer.

Calculation per m2.

m2       500.00 x____________ =____________

----------------------------------------------------------------------------------------------------------------------

06.009  Dismantling of existing concrete curbs iwhere is designed

new pipeline, temporary storage and placement of curbs

after pipeline installation and backfilling.

Curbs to place on concrete base made of MB15 concrete

(0.05m3 of concrete per 1m' of curb).

Unit price covers all necessary work and proper

placement of curbs (earth works, concrete works, transport...).

Calculation per m' of built in curbs.

m'       300.00 x____________ =____________

____________________________________________________________________________________________

Other works

Predmer i predračun                   Sewage sewer system in Berane                                        Strana  18

=======================================================================

Poz.    Opis pozicije                                                     J.M.     Količina    Jed. cena        Ukupno

=======================================================================

06.010  Dismantling of existing fence and doors made of iron, on

sections where it is necessary, temporary storage and

placement, after installing and backfilling of pipeline.

Unit price covers all necessary work, breaking with pickamer

existing concrete and stone walls, truck lod and transport

to landfill up to 10km distance, earth, carpentry, locksmith and

concrete works, as well as procurement and placement of

necessary material to bring fence in original state.

Calculation per m'.

m'       200.00 x____________ =____________

----------------------------------------------------------------------------------------------------------------------

06.011  Breaking of existing RC supporting walls and fences with

truck load and transport to landfill, up to 10km distance.

Concrete thickness from 20-50cm.

Unit price covers procurement with casting of MB30 concrete,

necessary concrete iron (round 100km per 1m3 of concrete),

formwork, as well as earth and other works.

After trench backfilling, it is necessary supporting walls and

fences to bring in original state.

Calculation per m3 of casted concrete.

m3        50.00 x____________ =____________

----------------------------------------------------------------------------------------------------------------------

Ukupno:                                                                                                      ____________

____________________________________________________________________________________________

Other works