- ANNEX II + III:	TECHNICAL SPECIFICATIONS + TECHNICAL OFFER

- Contract title: Supply of equipment for the Alignment with the EU Internal Market Acquis to Specialized Institutions of Montenegro	p 1 /…

Publication reference: EuropeAid/138711/IH/SUP/ME

Columns 1-2 should be completed by the Contracting Authority

Columns 3-4 should be completed by the tenderer

Column 5 is reserved for the evaluation committee

Annex III - the Contractor's technical offer

The tenderers are requested to complete the template on the next pages:

- Column 2 is completed by the Contracting Authority shows the required specifications (not to be modified by the tenderer),

- Column 3 is to be filled in by the tenderer and must detail what is offered (for example the words “compliant” or “yes” are not sufficient)

- Column 4 allows the tenderer to make comments on its proposed supply and to make eventual references to the documentation

The eventual documentation supplied should clearly indicate (highlight, mark) the models offered and the options included, if any, so that the evaluators can see the exact configuration. Offers that do not permit to identify precisely the models and the specifications may be rejected by the evaluation committee.

The offer must be clear enough to allow the evaluators to make an easy comparison between the requested specifications and the offered specifications.

Unless otherwise specified, the requirements in these Technical Specifications are presented as a minimum standard, which offers goods must meet.

1. GENERAL REQUIREMENTS

Below general requirement shall apply to all of the items specified.

1.1. Technical Requirements

    - All the equipment shall be provided complete with the necessary accessories and/or parts such as to ensure that the unit is capable of operating to the required technical and quality specifications.

    - All specification details listed for each item are the minimum requirements. Any improvements on the specifications or additional features offered should be clearly identified in the tenderer’s offer.

    - The tenderer should submit equipment brochures and catalogues showing the specifications of the equipment.

    - It should be noted that whenever a specific name of a product is mentioned in the Technical Specifications, a sufficiently precise and fully intelligible description is not possible, and it has to be understood as that product or its equivalent.

    - Wherever reference is made in the Technical Specifications to specific standards and codes to be met by the goods and materials to be furnished or tested, the provisions of the latest current edition or revision of the relevant standards in effect, shall apply, unless otherwise stated in the Contract. Where such standards and codes are national or relate to a particular region, other authoritative standards that ensure substantial equivalence to the standards or codes specified will be evaluated.

    - The type of the supplied voltage in Montenegro is 220 V (mono-phase) and 380 V (three-phase + neutral). The quality and stability of the supplied current may undergo fluctuations (+ and -) of more than 10%. All hardware must operate on 220 V ± 20 V, 50 Hz ± 0.5 Hz, power supply and be suitable for direct connection to the standard power outlets in Montenegro. The type of electrical outlets generally installed in Montenegro is the type with 2 side mounted earthling poles (Type F plug CEE 7/4). All plugs of all the supplied equipment shall fully comply with this standard.

    - The items (unless custom built) provided should not be hybrids and should be a registered brand name. Because of the variety of equipment, it is impossible to put the “one producer” condition, thus for compatibility purposes it is required to follow the compatibility requirements very strictly (PC compatibility, graphics compatibility, etc.). All proposed items must be fully compatible with each other.

    - The Supplier will resolve any compatibility or interoperability issues between software and hardware supplied.

    - All hardware units must be operable in an office environment with regards to following temperature, humidity and dust conditions:

- Operating temperature: 100 C - 350 C

- Relative Humidity: 20% - 80%

- Dust: 0.40 gr./m3

    - All items supplied conforming to the necessary CE regulation / norm must carry a suitable CE badge of conformity, permanently fixed to the machine where applicable.

    - The Tenderer shall prove that the equipment proposed is produced by a manufacturer who is accredited by ISO 9001:2000 or later or an equivalent European quality mark.

    - All hardware and software delivered as part of the proposed system solution must be brand new and in their original boxes. The Supplier should ensure that none of the items has any construction defects or defects in materials, which may result in the loss of operability.

    - All proposed software must be latest version. Software media (CD or DVD), related documentation, and licenses must be provided. All software shall be licensed to the institution to be delivered.

    - Equipment, which allows upgrading of capacities, shall be provided in such a way that installing additional capacity without discarding the already installed capacities can perform upgrades.

    - The proposed versions of all of the system components must be demonstrable at the submission deadline of this tender.

    - Technical responses to this Technical Specifications Document must be outlined and marked on the vendor documents (product data sheets, official vendor web documents, declarations, etc.).

    - The Tenderer shall offer units of the same brand and model for the full quantity required for each Item. In case of items comprised of more than one device, this applies at the device level.

- 1.2. General Software Specifications

    - All software versions should be English.

    - All proposed software licenses must have at least 3 years upgrade without any additional costs.

- 1.3. Technical Documents to be included

    - All products included in these TS shall be supplied with appropriate documentation (i.e. original set of operating and users’ manuals developed by the vendor, instructions for routine use and maintenance procedure) in English.

    - The obligatory documents for the provisional acceptance:

    - Original certificate of origin by the competent authority,

    - Report of Final test in the factory (quality control certificate), where applicable,

    - Calibration certificates as specified at the specification requirements of particular item, where applicable

    - Declaration of Conformity that the equipment is complying with EU Regulations and CE norms,

    - Warranty document of the manufacturer and/or the supplier,

    - The equipment or the materials manufacturers that are offered by the tenderer must be accredited by applicable ISO certification.

    - No provisional acceptance can be pronounced without the presence of the complete set of documents.

- 1.4. Warranty and Product Support Services

    - Warranty is as defined in Article 32 of the Special Conditions. The Supplier will be expected to provide the warranty services for all equipment. This warranty shall remain valid for a minimum of 1 year after the provisional acceptance.

    - The Supplier is responsible for the provision of “maintenance” and “repair” of the supplies within the warranty period without any charge. The warranty services shall include maintenance, troubleshooting, and spare parts backed up with technical hardware and software support.

    - All repairs and maintenance conducted under the warranty period must be undertaken by service personnel authorised by the relevant manufacturer for these tasks.

    - The Supplier shall guarantee the proper operation of the supplies. In the event of failure of the supplies' operability during the warranty period, the Supplier shall restore it, and in case of impossibility to do so, shall replace it with equivalent supplies in terms of the technical characteristics. The Supplier at his own expense shall perform the replacement.

    - Service requirements:

    - In the event that any component necessary for running the system does not operate for 24 hours, the Supplier shall immediately replace the defective equipment.

    - The Supplier is obliged to troubleshoot the problem not later than 24 hours following the receipt of the notification of the Contracting Authority about the failure of the equipment and/or software. In all cases the Supplier shall overcome the problem and fully repair and re-integrate, within the reasonable deadline set by the Contracting Authority. If the reparation of broken equipment/part is not possible, Contractor shall replace that equipment/part with another equipment/part. No additional cost will be borne by the Contracting Authority or the Beneficiary.

iv.     If any component of the system turns out to be defective during the warranty period, within less than 90% of the stated period of mean-time-between-failures, the Contractor shall replace this component with another one possessing the same technical characteristics and quality without any additional expenses to the Contracting Authority.

v.     The Supplier shall provide the Beneficiary with the telephone numbers active from Monday through Friday (7:00 a.m - 3:00 p.m., local time) for notification about the failures and reclamations.

vi.  Suppliers, in order to ensure prompt elimination of the potential defects, must demonstrate availability of service for all the supplied equipment.

    - The Supplier shall provide or secure the provision of a reliable and regular maintenance service to guarantee maintenance as well as the rapid replenishment of spare parts and consumables during the Warranty Period.

- 1.5. Delivery and installation of the equipment

    - The Supplier shall transport, unload, unpack, assemble, install, start-up, and configure the equipment, and also install the necessary data and electrical cabling in order to deliver the equipment in fully operational condition.

    - Delivery addresses for the equipment are provided below. The Supplier shall agree the installation schedule with the Contracting Authority and the Beneficiary. The Beneficiary shall guarantee the Supplier’s free entrance to the buildings and relevant room(s) during the installation works. The Supplier shall be fully responsible for its equipment until the provisional acceptance has been issued.

    - The equipment of this tender will be used in office and laboratory environments. The Contractor in coordination with the staff of the delivered institution should install all equipment.

    - The Supplier, after installing and configuring the systems and rest of equipment, shall perform all necessary tests to ensure the operational readiness of the installed equipment in order to deliver them in full functionality.

    - The Supplier must deliver and install the Supplies in an orderly and tidy manner. Power cords and data cables, including patch cords for all networked devices, shall be run cleanly and efficiently between the equipment and power sockets and shall not cause undue obstruction or in any way create a safety risk or health hazard. Under no circumstances should cables run over the front edge of a desk/bench into a walkway. Where excess cable cannot be removed, the appliance must not be used with the excess cable coiled.

  - Training Requirements

    - If necessary, the Supplier shall provide training materials. The language of the training and the training documents will be in Montenegrin; if not, interpretation/translation will be provided and paid by the Contractor.

    - The Supplier shall provide adequate trainings for a number of end users as specified in the below table in a way that they will be able to install, configure, administer and maintain the equipment and system without any help.

The lot/item numbers, which the training is needed, minimum number of end users to be trained and the minimum period of the training are given in details below.

| Item Number | Minimum Training Requirement |
| --- | --- |
| 1 and 2 | At least 2 staff of the beneficiary will be trained for 3 days on Item 1.1 and 3 days on Item 1.2 |
| 3 | At least 2 staff of the beneficiary will be trained for 3 days on this Item 1.3. |
| 4 | At least 2 staff of the beneficiary will be trained for 5 days on this Item |
| 5 | At least 2 staff of the beneficiary will be trained for 2 days on this item |
| 6 | At least 2 staff of the beneficiary will be trained for 2 days on this item |
| 7 | At least 2 staff of the beneficiary will be trained for 4 days for this item |
| 8 and 9 | At least 2 staff of the beneficiary will be trained for 2 days on item 8 and 3 days on item 9 |
| 10 and 11 | At least 2 staff of the beneficiary will be trained for 5 days for both items |
| 12 and 13 | At least 2 staff of the beneficiary will be trained for 2 days for both items |
| 16 and 18 | At least 2 staff of the beneficiary will be trained for 2 days for both items |
| 19 | At least 2 staff of the beneficiary will be trained for 2 days on this Item |
| 23 and 24 | At least 3 staff of the beneficiary will be trained for 5 days (item 23), and 3 staff trained for 3 days (item 24) |
| 25 | At least 8 staff of the beneficiary (Sanitary inspection) will be trained for 1 day |
| 27 and 28 | At least 10 staff of the beneficiary (Market inspection) 1 day for both items |

Training will be carried out at the delivery places of the equipment and it will cover training in instrument operation, including operation tasks.

The Supplier must include in his offer the methodologies for the training.

This offer shall include:

- The type of training

- Hours/day offered for each item

- Qualifications of trainers

- Description of methodology

Starting date and timetable of the training courses should be determined by mutual agreement between the beneficiary and the supplier.

  - Visibility

    - The Supplier shall take the necessary measures to ensure the visibility of the European Union financing and co-financing. These measures must comply with the rules laid down and published by the Commission on the visibility of external actions.

    - All equipment shall be labelled in accordance with the Communication and Visibility Manual for European Union External Actions.

Supplies shall be delivered at the following addresses:

| Item number | Institution | Delivery Address |
| --- | --- | --- |
| 1-9 | BUREAU OF METROLOGY | Kralja Nikole 2, 8100 Podgorica |
| 10-22 | BUREAU OF METROLOGY | Kralja Nikole 2, 8100 Podgorica |
| 23-24 | AGENCY FOR MEDICINES AND MEDICAL DEVICES (CALIMS) | Ivana Crnojevića 64a, 81000 Podgorica |
| 25-26 | ADMINISTRATION FOR INSPECTION AFFAIRS | Oktobarske revolucije 130, 81000 Podgorica |
| 27-28 | ADMINISTRATION FOR INSPECTION AFFAIRS | Oktobarske revolucije 130, 81000 Podgorica |
| 29-30 | INSTITUTE FOR STANDARDIZATION (ISME) | VII Omladinske 28, 81000 Podgorica |
| 31-35 | BUREAU OF METROLOGY | Kralja Nikole 2, 8100 Podgorica |
| 36-44 | ACCREDITATION BODY (ATCG) | Džordža Vašingtona 79, 81000 Podgorica |

LABORATORY EQUIPMENT FOR ELECTRICAL, TIME AND TEMPERATURE MEASUREMENT

| 1. Item Number | 2. Specifications Required | 3. SpecificationsOffered | 4. Notes, remarks, ref to documentation | 5. Evaluation Committee’s notes |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 1. | Universal meter testing equipment designed for the simultaneous testing of 1 to 20 meters with additional features to perform various type tests Quantity: 1 - Accuracy of the complete system: < 0.008 % - One power supply unit - Frequency Waveform Generator: Test circuit frequency: computer controlled adjustable to f=40 to 70 Hz; Test voltage circuit: selectable in the range of 40 to 320 V (phase-neutral) and 70 to 550 V (phase-phase); Test current circuit: selectable in the range of 1 mA to 120 A; Phase position adjustment to any power factor cos ᵩ / sin ᵩ = 0.000 to 1.000 in all 4 quadrants, computer controlled - One test voltage amplifier: Output power in the test voltage circuit: max. 3x500 VA - Three Test current amplifiers: Output power in the test current circuit: max. 2800 VA - Reference standard: For Active, Reactive and Apparent Power DC capable connected directly to the primary test circuit measuring modes - 1 Phase 2 wire active/reactive mode - 3 Phase 3 wire active/reactive modes true or cross-connected - 3 Phase 4 wire active mode/reactive mode true or cross-connected - Test frequency - Fundamental frequency range 40 to 70 Hz - Total detectable frequency - Test frequency - Fundamental frequency range 40 to 70 Hz - Range 0 to 3500 Hz - Measuring range - Voltage 30 to 500 V (phase-neutral) in the range: 480V/240V/120V/60V - Current 1 mA to 160 A, in the ranges: 200A /100A /50A /20A /10A /5A /2A /1A /500mA /200mA /100mA /50mA /20mA /10mA/5mA - Five isolating current transformer system: three-phase Isolating current transformer system provides the testing for single-phase and three-phase electricity meters accuracy (no additional error for reactive energy) - voltage/current < 0.005 % - power energy(active/reactive) <0.008% at cosφ=1 or sinφ=1 < 0.02% at cosφ=0,5 or sinφ=0,5 < 0.04% at cosφ=0,25 or sinφ=0,25 - phase angle < 0.005° High frequency output (60 kHz nominal) - Application software: - Compatible and based on Windows 7 or equivalent, - Network-compatible and bases on SQL Databases - Includes import and export features for test processes and test results - Provides comprehensive statistics features for quality control - Consists of a number of programs, all capable of running on their own - Provides a user interface for system control - Designed for generation and control or automatic test procedures - Displaying the test results of the meters under test. - Measuring system: perform error calculation. Designed for the pulse processing of: - Power proportional pulses of the reference standard - Scanning head pulses - Pulses of pulse transmitters - IR-data heads: 20 infrared scanning heads; read-out baud rate 9.600 Bd Max. 30kB - Test bench: - 2 stationary racks for 20 meters, each table designed for 10 meters - The connection of the meters will performed via quick acting devices - Pins are flexible and adjusted to all common single phase as well as three phase meters - The pins are chargeable up to 60 Amps continuously and 120 Amps for short periods (t < 2 min.) The scanning system with the following characteristics: - 20 optoelectronic scanning heads for the detection of rotor marks of inductive type meters and LED pulses of electronic meters - Suspension of the scanning heads at individual carriage - 1 Set of switches and sockets - 1 Set of cables - With calibration certificate from NMI signed of MRA agreement and CMC in BIPM Data Base Appendix C |  |  |  |  |  |
| 2. | Network time server Quantity: 2 - Selectable Reference Sources: PPS, 10 MHz, GPS, PTP/IEEE1588, NTP, Timecodes - IRIG/AFNOR timecodes (AM/DCLS) - The priority of all input signals configured - Requests per second: up to 10,000 NTP - Supported modes: - NTP Clients (IPv4, IPv6) - NTP Servers (IPv4, IPv6) - PTPv2 (IEEE 1588-2008 - MD5 Authentication and Autokey Key Management - Network interferace: at least 2 x 10/100 MBit with RJ45 connector and 2 GbE network interfaces with RJ45 connector - Reference Inputs: - GPS antenna input - PPS input - 10 MHz input - Time Code input - Signal Outputs: RS232 interface, PPS, 10 MHz - Ultra-stable integrated oscillator OCXO-SQ and integrated Rubidium PRS-10 - Accuracy of pulse outputs :< ±100 ns - Form Factor :Rackmount 3U chassis for standard 19'' racks |  |  |  |  |  |
| 3. | Frequency counter Quantity: 1 - Measurement range: DC to 3 GHz - Input: At least one input for BNC connector (Input impedance: 50 Ω or 1 MΩ, switchable) - Resolution: 10-digit - External reference input/outputs: 10 MHz via BNC connector With calibration certificate from NMI signed of MRA agreement and CMC in BIPM Data Base Appendix C |  |  |  |  |  |
| 4. | Maintenance bath (Water triple point) Quantity: 1 - Temperature range: ± 0,01 ºC to ±0,03 ºC - Accuracy: ±0,001 ºC, ±0,0001 ºC in cell - Ambient limits: 18 ºC to 28 ºC - Options: Triple point of water cells - For type A cells |  |  |  |  |  |
| 5. | Instrument for Environmental Data (Air Temperature and Humidity) Quantity: 1 - Memory: at least 2 mio. Measurement values - Measurement range: (-20 to +70ºC), Ctd 0 to 100% RH - Uncertainty: ±0,3ºC or better (-20ºC to + 70ºC) - Resolution: 0,1ºC; 0,1 RH - Battery life: at least 3 years at 15 mins measurement rate - 2 channels: 1 x internal for measuring ºC; 1 x internal for measuring % RH. |  |  |  |  |  |
| 6. | Portable Humidity and Temperature Generator Quantity: 1 - Control range: 5% RH-95% RH and 0ºC-70ºC - Control stability: <0,02ºC, <0,1% RH - Temperature gradient: ± 0,15ºC (et 0ºC); <0,05ºC (15-60ºC) - Humidity gradient: 0,1% RH or better - Time to set-point: 3 minutes to 5 minutes - Temperature sensor: Pt 100 - Humidity sensor: capacity RH sensor - Accuracy control probe: - ± 0,8% RH (et 23ºC±5)/ ±1,5% RH (0ºC-70ºC) - ± 0,1ºC (et 23ºC±5)/ ±0,2ºC(0ºC-70ºC) - Uncertainty: ±1,5% RH et 23ºC; 0,15ºC - External interface: USB connection, RS 232 communication |  |  |  |  |  |
| 7. | Dew Point Hygrometer Quantity: 1 - Measuring ranges: Frost/Dew point: -20ºC-70ºC Relative humidity: 5% rh-100% rh - Temperature (head mount PRT) - Temperature (cable mount PRT) - Accuracy: Frost/Dew point: ≤ ± 0,1 °C Temperature: ≤ ± 0,07ºC - Reproducibility: Frost/Dew point: : ≤ ±0,05ºC Temperature: : ≤ ± 0,05ºC - Temperature probe: PRT, 100 mm and et least 2 m cables - Mirror temperature sensor: Pt 100 - RS 232 communication - With calibration certificate from accredited laboratory traceable to NMI signature of MRA and CMC in BIPM Data Base Appendix C. |  |  |  |  |  |
| 8. | Precision Multimeter Quantity: 1 Resolution: 8.5 DIGIT Range: DC Voltage: from 0 V up ±1025V with best Best 1 Year Total Accuracy equal or better than ± 4 ppm AC Voltage: from 20mV up to 1025V, 10Hz - 1 MHz with Best 1 Year Total Accuracy equal or better than 0.015% DC Current: from 0 A up to ± 30A with Best 1 Year Total Accuracy equal or better than± 7 ppm AC Current: from 0µA up to 30A with Best 1 Year Total Accuracy equal or better than 0.03% Resistance: from 0 Ohms up to 1 GOhms with Best 1 Year Total Accuracy Accuracy equal or better than 8 ppm Resistance (electrometer): from 10 MOhms up to 1 TOhms, 10V - 300V with Best 1 Year Total Accuracy equal or better than ±140ppm Frequency: from 10Hz up to 10MHz with Best 1 Year Total Accuracy equal or better than 5ppm Temperature (PRT / RTD): 2,3&4 Wire; Support for IEC751&ITS90 coefficients Temperature (thermocouple) Minimum 8 types |  |  |  |  |  |
| 9. | Transportable Multiproduct Calibrator Quantity: 1 Range: DC Voltage: from 0 V up to ± 1000V with Best 1 Year Total Accuracy equal or better than ± 25 ppm AC Voltage: from 20mV up to 1000V, 10Hz - 500kHz with Best 1 Year Total Accuracy equal or better than 0.04% DC Current: from 0 A up to ±30A with Best 1 Year Total Accuracy equal or better than ± 50 ppm AC Current: from 0µA - 30A, 10Hz - 10kHz with Best 1 Year Total Accuracy equal or better than 0.06% Resistance: from 0 Ohms up to 1 GOhms with Best 1 Year Total Accuracy equal or better than ± 40 ppm Capacitance (passive) from 10nF up to 10uF with Best 1 Year Total Accuracy equal or better than 0.25% Frequency: from 1Hz up to 10MHz with Best 1 Year Total Accuracy equal or better than 20 ppm |  |  |  |  |  |
| 10. | Equipment for determination of silver jewellery alloys-Volumetric (potenciometric) method using sodium chloride or potassium chloride according to ISO 13756:1997 (Titrator with the appropriate user software and laptop computer) Quantity: 1 - The titration unit, which ensures measurement of mass fraction of silver in precious metal alloys using potentiometric titration of the sample AgNO3 in NaCl solution, in accordance with standardized procedures of ISO 13756,with auto sampler. - Titration unit includes an automatic burette (V = 20 ml to 25 ml) into the tank with the titrant solution. - The volume of droplets in the equivalence point must be less than 0.05 ml. - Option for automatic flushing of the sediment on the electrodes must be enabled. - Ag working electrode Reference electrode: mercury sulphate electrode or alternatively Ag / AgCl. Note: working and reference electrodes can be combined with a combined Ag electrode. - Mixer with propeller extension for solution mixing •Autosampler with at least 10 places - compact dimension of autosampler with control from titrator - cup volume 100 ml for sample - diafragm pump for cleaning of hard dirty samples - RinseUnit Power ShowerTM - Rinse tube to connect the diap - Disposable titration beakers - User software with laptop computer: Three core processor 2410M 2.3GHz (2.9GHz Turbo Mode, 3MB L3 Cache), 4GB DDR3 (4GB DDR3 1333MHz x 1), 500GB SATA 7200rpm, or equilvalent, 13 "LED-backlit HD + (1600 x 900), AMD Radeon HD 6470M 512MB DDR3, Microsoft Windows 64-bit compatible, or equivalent - Connecting cables for communication between titration unit and computer, as well as cablesfor connecting the electrodes. Operating instructions should be in English Instalation with basic education |  |  |  |  |  |
| 11. | Fume Cupboard Quantity: 1 - With EN 14175 certificate - With hot acid-resistant (ceramic) lining suitable for work with: - Organic solvents, - Weak acids, - Mineral acids, - Cold concrentrated mineral acids, - Hot concetrated mineral acids and - High thermal load (when boiling strong mineral acids such as nitric acid, hydrochloric acid, sulphuric acid...) - Useful dimension (mm) – WxDxH: 1123x600x660 - Overall dimension (mm) – WxDxH: 1185x760x1120 - With acid-resistant ventilator with fire protection and appropriate volume extraction in relation to the dimensions of the laboratory (5200 mm x 4400 mm x 3000 mm) and high thermal loads produced during the work. - With acid resistant water installation and electrical supply sockets with automatic protection and automatic fuses 230V-50H: 230V-50Hz - With water and acid resistant lighting in the working space – lux >1000 - With all other instalations including acid resistant ventilation pipes of sufficient length neccesary for proper work of fume cupboard in the place of use. - With acids and basesunderbenchcabinet (for nitric, hydrochloric acid,...) made of appropriate material suitable for installation under the fume cupboard and connected on the fume cupboard ventilation - Noise level - dbA<52 - Working Aperture – 200 mm - Exhaust flow rate m3/h – 600 - Mobile Stand - Operating instructions should be in English |  |  |  |  |  |
| 12. | Balance for control of pre packaged products, capacity 8kg Quantity: 1 • Maximum capacity: 8.1 kg • Readability: 0,01 g • Weighing pan dimensions (DxW): 172 mm x 205 mm • Linearity ±: 20 mg • Legal trading: Yes • Repeatability (typical): 4 mg Beyond weighing Accurate analytical results Instalation with basic education |  |  |  |  |  |
| 13. | Balance for control of pre packaged products, capacity 16kg Quantity: 1 -Maximum capacity: 16,1 kg -Readability: 0,1 mg -Repeatability (typical): 40 mg -Minimum weight (USP), typical 82 g -Weighing pan dimensions (DxW): 280 mm x 360 mm -Linearity ± 200 mg -Repeatability (Test Weight): 80 mg (5 kg) -Settling time: 1.5 s -Dimensions (DxHxW): 412 mm x 122 mm x 360 mm -Adjustment: Internal /proFACT advanced -Linearity: 200 mg Instalation with basic education |  |  |  |  |  |
| 14. | Printer for balance for control of pre packaged products Quantity: 2 - Printing Technology: Dot matrix - Printing Speed: Up to 10 mm/s - Printing Material: Standard paper (paper, continuous label) - Printing Languages: English, German, French, Italian, Spanish, Russian, Chinese (simplified), Japanese - Interfaces: Ethernet; RS-232; USB Device - Barcode Generation: No - Fast and error-free documentation - Print on paper and labels - Print on paper or continuous label for full documentation of results - When regulations require dot matrix |  |  |  |  |  |
| 15. | Transport case for balance Quantity: 1 - Dimensions (DxHxW): 285 mm x 345 mm x 535 mm - Balance Transportation -Protect of balance from damage during relocation - Platform precision balances with readability 5mg/10mg/0,1 g with printer and Ergostand. - Theft protection - Each case has an individual lock and key to transport your balance with absolute safety. |  |  |  |  |  |
| 16. | Software for balance for control of pre packaged products – MT FreeWay Net Compact Quantity: 1 - Databank for items with: - name, number and control number - nominal weight or volume - mean value or separate tare for each sample - control method through single or additive weighing system - number of samples in each control batch - Reports on product statistics by hour; shift; day; week; month; year. - Possibility of obtaining numerical or graphical reports - Data protection - Protection through coded system data entry All check weighing results are available in the computer and can be printed at any time with graphical and numerical charts, sorted by purchaser, product, date, time etc. |  |  |  |  |  |
| 17. | Portable density meter Quantity: 1 - -Measurement range: 0 – 2 g/cm3 - -Resolution: 0,0001 g/cm3 - -Temperature compensation: Automatic or user-defined - -Complete kit with accessories and transport case |  |  |  |  |  |
| 18. | Pycnometer 100 ml marked on 3rd decimal place Quantity: 1 Pycnometer volume 100 mL±3 mL with a marked volume on the flask of the pycnometer to the third decimal. Made of high quality borosilicate glass. With the volume determination form obtained from internal control laboratory (NMI or accredited laboratory) from which an accurate pycnometer volume and all conditions in laboratory pycnometer calibration are visible, through the factory calibration certificate and factory glass quality certificate. Each pycnometer (glass stopper and flask) is marked with a unique serial number (e.g. 001/2015, 002/2015 etc.) related to the above mentioned certificates. |  |  |  |  |  |
| 19. | Pycnometer 50 ml marked on 3rd decimal place Quantity: 1 Pycnometer volume 50 mL±3 mL with a marked volume on the flask of the pycnometer to the third decimal. Made of high quality borosilicate glass. With the volume determination form obtained from internal control laboratory (NMI or accredited laboratory) from which an accurate pycnometer volume and all conditions in laboratory pycnometer calibration are visible, through the factory calibration certificate and factory glass quality certificate. Each pycnometer (glass stopper and flask) is marked with a unique serial number (eg. 001/2015, 002/2015 etc.) related to the above mentioned certificates. |  |  |  |  |  |
| 20. | Pycnometer 100 ml marked on 2nd decimal place Quantity: 1 Cylindrical pycnometer volume 100 mL difference between the nominal and actual volume ±5 mL. With a marked volume on the flask of the pycnometer to the second decimal. Made of high quality borosilicate glass. Each pycnometer (glass stopper and flask) is marked with a unique serial number (eg. 001/2015, 002/2015 etc). With calibration certificate issued by NMI or accredited laboratory |  |  |  |  |  |
| 21. | Pycnometer 50 ml marked on 2nd decimal place Quantity: 1 Cylindrical pycnometer volume 50 mL difference between the nominal and actual volume ±53 mL. With a marked volume on the flask of the pycnometer to a second decimal. Made of high quality borosilicate glass. Each pycnometer (glass stopper and flask) is marked with a unique serial number (eg. 001/2015, 002/2015 etc). With calibration certificate issued by NMI or accredited laboratory |  |  |  |  |  |
| 22. | Pycnometer 290/IV Quantity: 1 •Stainless steel material in col. with ISO 2811 •Metal pycnometers volume 50 and 100ml •ICP cover •Lid opening set •With calibration certificate issued by NMI or accredited laboratory |  |  |  |  |  |
| Agency for Medicines and Medical Devices -CALIMS | Laboratory for detection of substandard and falsified medicines (High Voltage generator, X-ray sources, Goniometer, Sample platform, Automatic sample changer, Incident beam modules, Fast detector, Software). |  |  |  |  |  |
| 23. | Modular Multipurpose Powder Diffraction System Quantity: 1 - High Voltage Generator: Power 3 kW or more. - X-ray sources: Long Fine Focus sealed ceramic X-ray tubes with Cu, anode. X-Ray tube and tube housing allowing easy change from line to point focus and vice versa without any realignment supplies re-connection or special tools. - Goniometer: Vertical Theta/Theta goniometer, with mechanism for fast exchange of all optical modules and sample stages without need for any re-alignment after exchange. Angular range at least: -110<2Theta< 164 degrees. The smallest step: 0.0001 degree or less. Sample platform: Rotational sample stage (sample “spinner”) with programmable rotation speed and programmable angular position, ready for operation in reflection and foil transmission mode, suitable for following sample types: standard reflection (“back” or “front”); Si single crystal holders (zero background) for small amount of sample and holders suitable for loading small irregularly shaped solid samples. The sample stage should be exchangeable with other sample stages without the need for any alignment. - Automatic sample changer: Automatic sample changer allowing automatic loading of min. 15 samples in one single automatic batch with sample holders. - Incident beam modules: - Incident beam module for monchromatic radiation allowing standard Bragg-Brentano mode of operation, suitable also for SAXS and thin layers analysis, -Focusing X-ray mirror with minimum length of the crystal of 50mm, with the focusing point at the detector plane for measurement in transmission. - Fast Detector: Solid state fast 2D detector. Detector allows scaning and static measurements in «point detector» mode («0D»), linear mode («1D») and area mode («2D»). Pixel size should be <=70 micron, Number of pixels 256x256 or more. - Software: (license for 10 interdependent installations) Instrument control and experiment setup, General data treatment, Qualitative and (semi) quantitative analysis; Crystallographic analysis, Data Clustering. - Computer and accessories: PC with LCD monitor and color printer. |  |  |  |  |  |
| 24. | Benchtop Energy Dispersive X-Ray Fluorescence (EDXRF) Spectrometer for Elemental Analysis. Quantity: 1 - The XRF spectrometer capable of analyzing elements in the periodic table, from Na to Am, from ppm to %. - Sample types: powders (loose and pressed), fused beads, liquid, films, large samples - Sample loading: sample changer with 10 or more positions, with the ability for sample rotation during the measurement - X-ray tube type: Ag anode X-ray tube. - X-ray tube power: 9 W or more - X-ray tube voltage/current: - Tube settings controlled by software - Max. voltage of the X-ray tube 50 kV or more - Max. current 1 mA or more - X-ray tube filters: six tube filters, software selectable. - Detector: High-resolution Si drift detector, energy resolution better than 145 eV. Computer: PC with LCD monitor and color printer |  |  |  |  |  |
| Administration for Inspection Affairs |  |  |  |  |  |  |
| 25. | Tube-based handheld XRF-analyser Quantity: 3 - Rhodium(Rh) thin window X-ray tube based 50 kV handheld XRF analyzer 15-50 kV - Automated 5 position primary beam Filter changer with 4 pre-fitted filters optimized for Alloys, Restricted Materials, Plastic and Geo - High Performance SiPIN detector with elemental range Cl – U - • Protected against detector puncture with the patented TITAN Detector Shield (TDS) - Hot Temperature (up to 450 deg.C) measurement capable with optional KAPTON(TM) window - Light weight, <1.5 kg with batteries for increased productivity by reducing operator fatigue - Features the patented SharpBeam™ optical beam path for best performance at lowest power settings extending battery life and avoiding overheating as well as defined beam spot of 5mm with no movable parts. - IP54 rated industrial grade molded housing with no-slip rubberized grip and contour housing protection • Should include at least 2 high performance Li-Ion batteries with ventilated smart charger • With minimum 4 GB USB stick for data Storage and reporting and toolbox software for instrument - Waterproof with pressure equilibrated transport case - Instrument weigh of 1.5kg or less - Alloy calibration package for heavy elements only (Ti-Bi). Should include specific calibrations for stainless steels, tool steels, low alloy steels, cobalt alloys, nickel alloys, copper alloys, zinc alloys, light alloys and general calibration for all other alloy types including alloy check sample. |  |  |  |  |  |
| 26. | Standard Weight Set with certificate Quantity: 1 - Range (1 mg-1kg) - OIML: Accuracy F2 - Stainless steel 7,90-7,95 kg/dm3 - Standard weight set at series: 1,2,2,5 - In service case Quantity: 1 - Range (1 mg-5kg) - OIML: Accuracy M1 - Standard weight set at series: 1,2,2,5 - Stainless steel 7,90-7,95 kg/dm3 - In service case Quantity: 1 - Range 10kg - OIML: Accuracy M1 - Stainless steel 7,90-7,95 kg/dm3 - In wooden box Quantity: 1 - Range 20kg - OIML: Accuracy M1 - Stainless steel 7,90-7,95 kg/dm3 - In wooden box |  |  |  |  |  |
| 27. | Universal measuring instrument for testing electrical safety Quantity: 3 Universal Test Instrument for DIN EN 60601/60335/60950/ 61010, DIN VDE 0700/0701-0702 and IEC 62353 (VDE 0751-1) Description: enables testing of the following technical requirements: measurement of current voltage; resistance; frequencies; capacity; temperature; remembering the MIN and MAX values; ground test; insulation test; processing and loss of energy; decrease of voltage on exposed conductive parts; contact current for SC2 devices; protective conductor for SC1 devices; resistance of insulation; equivalent losses and other tests according to EN 60335 standard. - Tester version: M7010- B01-C01-D21-F02-G01-J01-KA01-KB01-KE01-P00) with a EU variant (test socket and mains plug), English menu; test current, conductor 25A / 200 mA DC, power cable,interchangeable;a test cable on a 2 meter cable, a crocodile clip, DAkkS - a caliberbcertificate, with instructions for use. - Bag for carrying instruments and accessories, large; - Printer , memory RAM 100 kByte up to 300 tests, depending on the scope of master data,interface (I). - Accessory for the test instruments |  |  |  |  |  |
| 28. | TRMS Multimeter with analoge bargraphwith multi measuring functions: temperature measurement and infrared interface, with added frequency and capacitance measurements Quantity: 3 - Resolution: 100 μV, 100 mΩ, 10 μA, 10 pF, 0.1 Hz - Precision temperature measurement (–50 ... +800 °C) - Frequency and duty cycle measurement at 2 to 14 V signals - up to 1 MHz - Capacitance measurement - RPM Measurement with Inductive Sensor - Automatic and manual measuring range selection - Backlit digital display with additional analog scale - Measured value memory, - Hold, Max-Min value - Overload and blown fuse indicators - IP 40 protection - Automatic Blocking Sockets (ABS) - Automatic / Manual Measuring Range Selection - Display of Negative Values at the Analog Scale - Storage of Measured Values - convincing technology with all voltage, resistance, current and temperature measuring ranges. - Protective rubber holster - current measurement - voltage measurement - measurement of resistance - Frequency measurement - measuring capacitance - measuring temperature - Memorising MIN and MAX values. |  |  |  |  |  |
| ISME |  |  |  |  |  |  |
| 29. | Software for Automatization Notification Process into CEN/CENELEC Database and web site upgrade Quantity: 1 - Software should provide automatized export data from the ISME database and uploading the same data to CEN/CENELEC database - Document Management System - Migration data possibility - Completed technical documentation must be available Web Site Upgrade Quantity: 1 - Upgrade ISME web site to be more operable and functional. - Upgrade web site to provide electronic sales - Migration data possibility Source codes and documentation in English |  |  |  |  |  |
| 30. | Hardware and Software for Upgrading Network Infrastructure Quantity 1: Server: - CPU: min. 2 x 6-core processor cache 2.5MB per core, at least 2GHz - RAM: min.2x16GB DDR4 2400MHz RDIMM ECC Dual rank (12 DIMM slots) - Hard memory: 8x 2TB 3.5" NLSAS 12Gbps 7.2k - Network 2 x 10/100/1.000Mbps - Redudant power - Installation software: - Virtualization software (open-source) - Operating system (open-source) - Mail server (open-source) - Firewall (open-source) - Network controller - File backup - Data migration - Technical documentation in English - Smart-UPS 1500VA/1000W, Input voltage range for main operations: 160 - 286V, Input voltage adjustable range for mains operation 151 - 302V, Topology Line Interactive, Output Connections (8) IEC 320 C13 (Battery Backup), Multi-function LCD status and control console, Smart Slot, USB, Tower. |  |  |  |  |  |
| 31. | NAS Server Quantity: 1 - Standalone or Rack type - Minimum 2xHot swappable tray - Minimum internal 2 x 3.5" or 2.5" SATA 6Gb/s, SATA 3Gb/s HDD or SSD - HDD 2 x 4TB SATA 6Gb/s 7.2k - RAID technology 0/1 - Wired network speed 10/100/1000 USB 2.0, USB 3.0 |  |  |  |  |  |
| 32. | Mini Desktop Computers Quantity: 7 - Small Form Factor - CPU: minimum 2Core/4Threads, min 3.90 GHz, min 3MB cache, CPU with the rating at Pass Mark CPU list of at least 5.800 (https://www.cpubenchmark.net/cpu_list.php - RAM: minimum 4GB DDR3 - GPU: Intel HD compatible or equivalent, output 2 monitors (HDMI, Display Port) - HD: minimum 128 SSD 2.5” - DVD-RW DL - 1xGigabitEthernet port - Keyboard/Mouse USB - OS Linux compatible |  |  |  |  |  |
| 33. | Multifunction Printer Quantity: 1 - All in One, printer-scanner-copier A3 - A4 black – min. 30 ppm, A3 black – min. 15 ppm - Piper size : A5 to A3 - Duplex print - DADF – duplex scanning capacity min. 50 pages - Printing quality min. Up to1200 x 1200 dpi - Scanner: A3, min. 600 x 600 dpi - Zoom : Reduction / Enlargement 25 to 400% - Memorry: min. 512 MB - Network color scenning document - Ethernet RJ45, USB 2.0 Touchscreen Display |  |  |  |  |  |
| 34. | Projector Quantity: 1 - DLP projector - Native minimum Resolution XGA, 1024 x 768 - Resolution Support VGA(640 x 480) to WUXGA_RB(1920 x 1200) - Native Aspect Ratio 4:3 - Light Source Lamp minimum 203W - Video Input: 2x HDMI , S-Video, 2x VGA D-sub, Composite (3x RCA) - Video compatibility: 1080p, 1080i, 720p, 576p, 480p, 576i, 480i, NTSC, PAL, SECAM - Serial (RS-232), IR Receiver – Remote control. Weight: Max 2.4 kg |  |  |  |  |  |
| 35. | Interactive White Board + implementation Quantity: 1 - size minimum 80” connection to PC – USB |  |  |  |  |  |
| 36. | Scanner Quantity: 1 - Type: ADF Network document scanner - Display: Colour TFT Touchscreen 10,1” - Sensor type: CMOS CIS sensor - Light source: RGB LED - Scanning speed: - Black-white 45ppm/90ipm - Colour 45ppm/90ipm - ADF: 60 lists - Optic resolution: 600 dpi - Scanning type: Simplex; Duplex - Interface: 10 Base T/100 Base TX/ 1000 Base T; 3 x USB 2.0 - input documents: wide: 50 – 216 mm length: 54 – 356 mm thickness: 27 - 209 gr/m² (with separation) and 27 - 255 gr/m² (single) - Max daily rate of scanning: 6000 pages. |  |  |  |  |  |
| 37. | Server Quantity: 1 - Rack-mount server, maximum 1U - Minimum 2 processors with minimum 8-core each, minimum speed 2.0 GHz, with minimum 20 MB cache - Memory: 64GB TruDDR4, supported max capacity 1.5 TB RAM, 24 slots for memory DIMMs, support for memory mirroring and memory rank sparing - Install drives Minimum 2 SSD discs of minimum 240GB capacity - Minimum 4 x 1 Gbps Ethernet ports - Minimum 2 one-port FC 16GB HBA - Redundant hot-swappable power supplies, with minimum of 550 W per unit Operating Systems (OS) Supported : Microsoft Windows Server, Red Hat Enterprise Linux, SUSE Linux Enterprise Server, VMware vSphere. |  |  |  |  |  |
| 38. | Entry-level storage Quantity: 1 - Rack mount system, 19” - 6 discs 900GB 10k SAS - “No single point of failure” system, minimum 2 active controllers - Minimum 4 GB cache memory per controller, able to be extended up to minimum 8 GB per controller - Support for SSD, SAS and NL SAS discs - Support for minimum 120 discs - Support for RAID 0, 1, 5, 6, 10 - Support for 1 Gbps iSCSI, 6 Gbps SAS, 8 Gbps Fibre Channel and 10 Gbps iSCSI/Fibre Channel over Ethernet linking of storage systems - Minimum 4 x 1 Gbps Ethernet and 8 x 8 Gbps Fibre Channel ports for the needs of direct servers linking (include SFP transiver) - Thin/Virtual provisioning for entire RAW installed capacity - Creating “point-in-time” copies of data - Support for automatic and dynamic multi-tiring of data using SSD discs - Support for replication of data to remote storage system - Support for the following operative systems: Windows, AIX, Linux, Solaris, HP-UX - Multipath software with the support for load Load-balancing and failover Web based GUI for storage system management |  |  |  |  |  |
| 39. | L2 switch type 1 Quantity: 1 - L2/L3 switching device – rackable, stackable, up to 4 devices inside one stack, 1U device - Interfaces: 24 ports 10/100/1000, auto-sense; minimum 4 separate SFP ports; serial console port; - Performance: Data rate minimum 128Gbps, Throughput minimum 95 Mpps wire-speed - Protocols: Supported 802.1D, 802.1ab,802.1w and 802.1Q, 802.1p; minimum 1024 VLANs; VLAN stacking support; Port and MAC based VLAN; oSupport for DHCP relay; support for SNMP, RMON; Jumbo frame support 9216 bytes; 802.1X support; L2 QoS, L3 QoS; oMinimum 16000 hardware MAC address per device support - IP routing IPv4: InterVLAN routing support; static routing, 4096 IPv4 hardware host routes minimum; RIP support; OSPF support; 1500 ARP entries minimum, oPort, VLAN and Router based ACL support; MAC limiting 802.1X support; - IP routing IPv6: Minimum 2000 IPv6 host routes supported in hardware oRIPng, OSPFv3, IPv6 static routing supported - Traffic mirroring: support, VLAN ilia ACL based; - Link aggregation support LACP; - Support for RIPv1/v2, OSPFv1/v2, IGMP v1/v2/v3, Protocol Independent Multicast (PIM), IEEE 802.1 Q-in-Q, Bidirectional Forwarding Detection (BFD), Virtual Router Redundancy Protocol (VRRP) and Virtual Router / VRF-Lite with licence upgrade - Management: Full featured CLI (console, SSH); web GUI interface •Rescue configuration support; rollback support for image and config file - Environment: Operating temperature range 0–45 °C, relative humidity operating 10% – 85% Support: Official device registration on manufacturer’s web site for free software and firmware updates for a period on at least 1 year. |  |  |  |  |  |
| 40. | Firewall Quantity: 1 - Statefull firewall, UTM device, VPN concentrator and complete routing device: - Integrated power supply, rackmount form factor; - Flash and Memory: 4GB DRAM, 8GB Flash storage, - Performance: Firewall IP throughput (IMIX type) 500 Mbps minimum, Application secure firewall throughput: min. 500 Mbps, Maximum concurrent sessions (IPv4 or IPv6) min. 64000 Concurrent sessions per second 5000 minimum, Unrestricted number of internal users, IPS (intrusion prevention system) throughput 200 Mbps, o IPS sessions minimum 16000 - Interfaces: Fixed I/O: 6 x 10/100/1000BaseTX Ethernet ports minimum, 2 x SFP ports minimum Console serial port Support for E1 and E3 through module addition, Minimum one miniUSB 3 port; - IPSEC VPN support: IPsec VPN tunnels min. 256, IPsec VPN throughput (large packets, 1400B): min. 300 Mbps, Support for MD-5, SHA-1 and SHA-2 authentication, Dynamic remote access VPN support, - Routing: Routing (Packet Mode 1518B traffic) PPS – 1500Mbps, Routing table size (RIB) minimum IPv4/IPv6 - 256000 routes OSPF: full OSPF and OSPF v3 support Static routing support, BGP: BGP support, BGP routing reflector supported IS-IS support MPLS support: RSVP, LDP, L2/L3 MPLS VPN, pseudo - wires, VPLS support IPv6 support, IPv6 routing included, OSPFv3 minimum - Virtualization: Min 16 security zones supported Virtual router support, minimum 32 VR supported oSupport for 1000 VLANs minimum; - Environment: Operational temperature: 0° to 40° C Non-operational temperature: -20° to 70° C Operating humidity 10% to 90% - QoS and VoIP support; Guaranteed bandwidth; Maximum bandwidth; DiffServ; - Management: full functional CLI (ssh, console), WebUI (HTTPS); - General support: NAT, PAT support; bridge groups support; L2 mode support (transparent firewall); L3 mode support (route); - Active/active failover configuration support; active/passive failover support; ASCII configuration support, Rescue configuration support, configuration rollback support, image rollback support - Application security supported via licence upgrade; - IDP/IPS capabilities supported via licence upgrade; - Content security services supported via licence upgrade. |  |  |  |  |  |
| 41. | UPS Quantity: 1 - Minimum 2 UPS with the following characteristics: - Rack-mount 2HE UPS - Minimum capacity 3000 VA (2700 W) - Line Interactive - Minimum 8 x C13 ports - Minimum 1 x C19 ports - Transfer Time 4ms typical : 8ms maximum - Input Frequency 40 - 70 Hz (auto sensing) - Input voltage range for main operations 160 - 286V - Input voltage adjustable range for mains operation 151 – 302V - Interface Port(s) USB LED status display with On Line : On Battery : Replace Battery and Overload indicators, Multi-function LCD status and control. |  |  |  |  |  |
| 42. | Rack Quantity: 1 - Server Rack 26 U Height, 19” - Height; 1300-1400 mm Width: 600-610 mm; Depth: 1000mm - EIA-compatible - Perforated front and rear steel doors - Lockable doors and side walls - Two 1U sidewall compartments for power distribution. |  |  |  |  |  |
| 43. | Software requirements Quantity: 1 - VMware vSphere 6 Essentials Plus Kit for 3 hosts (Max 2 processors per host) with support for 1 year included. Software must support following guest operation system: -Microsoft Windows Server 2016, - Microsoft Windows Server 2012, - Microsoft Windows Server 2008, Red Hat Linux, SCO Open Server 5 Quantity: 2 Microsoft Windows Server 2016 Standard or equivalent. |  |  |  |  |  |
| 44. | Notebook computers (2-in-1) Quantity: 10 - Processor: minimum 2Core/4Threads, min 3.90 GHz, min 3MB cache, CPU with the rating at PassMark CPU list of at least 5.800 (https://www.cpubenchmark.net/cpu_list.php - Display: 13.3-14" FULL HD 1080p LED Display Touch (2-in-1) - Memory: minimum 4GB DDR4 2400MHz up to 16GB - Storage: SSD minimum 256GB - Intel HD Graphics Controller compatible or equivalent - Connections: 1 x USB 3.0, 1 x USB 2.0, HDMI-out, Headphone/microphone combo jack, USB 3.0 with PowerShare, SD Card - Networking: Gigabit-LAN (10/100/1000MBit), Wireless (a/b/g/n/ac), Bluetooth 4.0 - Battery: 3-cell - Operating System: Microsoft Windows 10 Home 64Bit compatible or equivalent. Weight: Max 1,70kg |  |  |  |  |  |

The cost of training should be calculated in the price of each item per which the additional training is required.